# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTRONIC CIRCUITS 								       Time: 3 hours

# Subject Code:	EE249		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the need for voltage regulators?

2.	Define percentage regulation of a rectifier.

3.	State the expression for output of a differentiator.

4.	What is a pulse transformer?

5.	Why is the input impedance of FET more than that of a BJT?

6.	State the causes of instability in DC amplifiers.

7.	_________ is defined as the ratio of maximum collector dissipation power to the maximum ac power developed across the load.

8.	_________ amplifier is a circuit in which a pair of NPN and PNP transistors conducts alternatively.

9.	_________ oscillator uses both positive and negative feedback.

10.	What is the frequency of oscillation of Astable Multivibrator?

PART – B (5 x 3 = 15 MARKS)

11.	What are the disadvantages of shunt capacitor filter?

12.	How will you choose the components of a clamper?

13.	Write short notes on Darlington Amplifier.

14.	What is the need for impedance matching in a power amplifier?

15.	What are the conditions for sustained oscillation?

PART – C (5 x 15 = 75 MARKS)

16.	A 230 V, 60 Hz voltage is applied to the primary of a 5:1 step-down, center-tap transformer used in a full wave rectifier having a load of 900 Ω.  If the diode resistance and secondary coil resistance together has a resistance of 100 Ω, determine (a) d.c. voltage across the load, (b) d.c. current flowing through the load, (c) d.c. power delivered to the load, (d) PIV across each diode, and (e) ripple voltage and its frequency.

(OR)

17.	Explain the operation of a Switched Mode Power Supply using a block diagram and suitable waveforms.

18.	Discuss the response of Low Pass RC circuit for ramp and exponential inputs.

(OR)

19.	Explain the different types of Biased Clipper using neat circuit diagrams and waveforms.

20.	Discuss small signal analysis of CE amplifier with Voltage-Divider Bias.

(OR)

[P.T.O]

21.	A CE, RC coupled amplifier uses transistors with the following h-parameters: hfe = 50, hie=1200 Ω, hoe = 30 x 10-6 mhos, hre = 2.5 x 10-4.  The value of gm at the operating point is 50 m mhos.  The biasing resistor R1 between VCC and base is 100 kΩ and R2 between base and ground is 10 kΩ.  The load resistor RC = 5 kΩ. Let C = 160 pF be the total shunt capacitance in the input circuit and the coupling capacitor CC = 6 µF.  Calculate for one stage of the amplifier (a) mid-band current gain (b) mid-band voltage gain (c) lower and higher 3 dB frequencies and (d) gain-bandwidth product.

22.	Explain the circuit operation, advantages and disadvantages of a Push-Pull Amplifier.

(OR)

23.	Describe the effects of negative feedback in amplifiers.

24.	Explain the operation of RC Phase Shift Oscillator using the equivalent circuit diagram.

(OR)

25.	Explain the operation of Schmitt Trigger using circuit diagram and waveforms.