# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DC MACHINES AND TRANSFORMERS 				       Time: 3 hours

# Subject Code:	EE250		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Brushes are normally made up of soft material like ___________.

2.	What is meant by separately excited generator?

3.	Write the voltage equation of a DC motor.

4.	What is the condition for maximum efficiency of a DC machine?

5.	What is voltage transformation ratio?

6.	What is the purpose of laminating the core of a transformer?

7.	How do you predetermine the efficiency and regulation of a single phase transformer?

8.	The saving in Cu achieved by converting a 2 winding transformer into an auto transformer is determined by ____________.

9.	Which 3 phase transformer connection is best suited for 3 phase, 4 wire service?

10.	What is the use of instrument transformers on AC circuits?

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between lap and wave winding for DC machines.

12.	Why is a series motor never started on no load?

13.	Derive the condition for maximum efficiency of a single phase transformer.

14.	What are the advantages of auto transformer?

15.	What are the conditions that have to be satisfied for the parallel operation of 3 phase transformers?

PART – C (5 x 15 = 75 MARKS)

16.	With a neat sketch explain the constructional features and working principle of a DC generator.

(OR)

17.	a.	Derive the emf equation of DC generator.                                         						(7)

b.	A 4 pole, DC shunt generator with a shunt field resistance of 100Ω and an armature resistance of 1Ω has 378 wave connected conductors in its armature. The flux per pole is 0.02 wb. If a load resistance of 10Ω is connected across the armature terminals and the generator is driven at 1000 rpm, calculate the load current and power absorbed by the load.

(8)

18.	a.	Derive the expression for torque on a DC motor.                             						(7)

b.	With a neat sketch explain the operation of 3 point starter used in DC shunt motor.            (8)

(OR)

19.	Explain the characteristics of series, shunt and compound motors.

[P.T.O]

20.	a.	Explain the operation of single phase transformer with load using phasor diagrams. 		(7)

b.	A 25 kVA single phase transformer has 500 turns on the primary and 50 turns on the 	secondary winding. The primary is connected to 3000V, 50 Hz supply. Find the full load 	primary and secondary currents, the secondary emf and the maximum flux in the core. 	Neglect leakage drops and no load primary current.                        						(8)

(OR)

21.	a.	Deduce the equivalent circuit of a single phase transformer.           						(7)

b.	A 100 kVA, 2 winding transformer has an iron loss of 1kW and a Cu loss on normal output current of 1.5kW. Calculate the kVA loading at which the efficiency is maximum and its efficiency at this loading at 0.8pf lagging.   											(8)

22.	a.	Prove that saving in copper in auto transformer is K times that of the total weight of copper 	in two winding transformer.                             										(7)

b.	Explain the open circuit test and short circuit test conducted in a single phase transformer.(8)

(OR)

23.	a.	With a neat sketch explain back to back test conducted in two single phase transformers.	(8)

b.	Explain about the polarity test conducted on transformers.                   					(7)

24.	Write short notes on

a.	Current transformer		b.	Potential transformer     						           (7+8)

(OR)

25.	a.	Explain the construction and working principle of three phase transformer.				(7)

b.	Explain any two types of 3 phase transformer connections with advantages and disadvantages.                                                         									(8)