# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	INDUCTION AND SYNCHRONOUS MACHINES 		       Time: 3 hours

# Subject Code:	EE251		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What type of magnetic field is produced in three phase induction motor?

2.	The magnitude of induced emf in the rotor also reduces by _________ times the magnitude of induced emf at standstill condition.

3.	How would you reverse the direction of rotation of a capacitor start induction run motor?

4.	Where do you place the starting winding of a single phase motor?

5.	Why are three phase alternators invariably Y- connected?

6.	When speed of an alternator is changed from 3600 rpm to 1800 rpm, what will be the generated emf/phase?

7.	Does change in excitation affect the synchronous motor power factor?

8.	What is a synchronous capacitor?

9.	What is the method used to determine Xd and Xq in a salient pole alternator?

10.	Draw power angle curve of alternator.

PART – B (5 x 3 = 15 MARKS)

11.	Define slip of induction motor. When induction motor is at rest, what is the value of slip?

12.	Is single phase induction motor self starting? Why?

13.	Differentiate salient and cylindrical type of rotor in alternator.

14.	State synchronous motor applications.

15.	An alternator has a direct axis synchronous reactance of 0.7 per unit and a quadrature axis synchronous reactance of 0.4 p.u. It is used to supply full load at rated voltage at 0.8 pf. Assume rated terminal voltage as 1 p.u and full load armature current as 1p.u. Neglect armature resistance. Find the total induced emf on open circuit.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the expression for running torque of three phase induction motor.				(8)

b.	Sketch and explain the typical torque-slip characteristics of a three phase induction motor.(7)

(OR)

17.	a.	Explain with neat sketch any two types of starters used in three phase induction motor. 	(8)

b.	List out various losses in three phase induction motor.                      					(7)

18.	Explain in detail about any two types of single phase induction motor with its applications.

(OR)

19.	a.	Develop equivalent circuit of a single phase induction motor ignoring core loss.  			(8)

b.	Write short notes on linear induction motor.                                        					(7)

20.	A 3 phase, 6 pole, star connected alternator revolves at 1000 rpm. The stator has 90 slots and 8 conductors per slot. The flux per pole is 0.05 wb (sinusoidally distributed). Calculate the voltage generated by the machine if the winding factor is 0.96. Derive the expression used.

(OR)

[P.T.O]

21.	a.	What are the conditions that have to be satisfied for synchronization of alternators. Explain 	any one method of synchronizing of alternators. 										(8)

b.	Find the synchronous impedance and reactance of an alternator in which a given field current produces an armature current of 200 A on short circuit and an generated emf of 50V on open circuit. The armature resistance is 0.1 ohm. To what induced voltage must the alternator be excited if it is to deliver a load of 100 A at a pf of 0.8 lagging with a terminal voltage of 200V.                                            													(7)

22.	Discuss the following:

a.	V curve		b.	Hunting                                                                                  		    (7+8)

(OR)

23.	Explain the principle of operation of synchronous motor and also discuss various methods of starting synchronous motor.

24.	Draw and explain the phasor diagram of salient pole alternator supplying full load current. Show that the power output per phase is given by

(OR)

25.	a.	Determine Xd and Xq of a salient pole alternator using slip test.     	 					(7)

b.	Calculate the synchronizing torque for one mechanical degree of displacement in 5.5 MVA, 3 phase, 50 Hz alternator when running at 1500 rpm with a generated emf of 9 kV. The machine has a synchronous impedance of 25%.                                                       		(8)