# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTRICAL MACHINE DESIGN 						       Time: 3 hours

# Subject Code:	EE252		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What are the main dimensions of a rotating machine?

2.	Define field form factor.

3.	What is meant by staggering of brushes?

4.	Which type of winding is used for high current, low voltage machines?

5.	How will you classify the transformer according to construction?

6.	Define Window Space Factor.

7.	Which type of induction motor will have high starting torque?

8.	Compare squirrel cage induction motor and slip ring induction motor.

9.	List the different types of alternator.

10.	What is runaway speed?

PART – B (5 x 3 = 15 MARKS)

11.	Define specific magnetic loading.

12.	What are the materials used for brushes in DC machines?

13.	List the different methods of cooling in Transformers.

14.	Define dispersion coefficient.

15.	Define short circuit Ratio (SCR).

PART – C (5 x 15 = 75 MARKS)

16.	Calculate the main dimensions of a 20 kW, 1000 rpm, 6-pole d.c. motor.  Given Bav  =  0.37 Tesla and ac = 16,000 ampere conductors per metre.  Make the necessary assumptions.

(OR)

17.	A 600 RPM, 50 Hz, 10000 V, 3 phase, and synchronous generator has the following design data.  Bav = 0.48 Wb/m2, δ = 2.7 A/mm2, slot space factor = 0.35, number of slots = 144, slot size = 120 x 20 mm, D = 1.92 m and L = 0.4 m.  Determine the kVA rating of the machine.

18.	The armature of a 10 pole, 1000kW, 500V, 300RPM, d.c generator has a diameter of 1.6m.  There are 450 coils.  Determine suitable axial length and diameter for the commutator, giving details of brushes, having regard to commutation conditions and temperature rise.  The design limitations are: peripheral speed of commutator 20m/s, pitch of segment 4 mm, current/brush 70A, temperature rise 40C. The other data given is:  The brushes span three segments approximately; brush contact drop = 1.5 V, coefficient of friction  = 0.15, brush pressure=20KN/m2, cooling coefficient = 0.012/(1+0.1Vc). Make suitable assumptions for clearance between brush boxes, staggering of brushes and end play.

(OR)

19.	a.	Derive the output equation of a d.c. machine.                                                     		(7)

b.	Prove number that the minimum of coils required in a dc machine armature is Ep/15.

(8)

[P.T.O]

20.	Derive the output equation of single phase transformer and three phase transformer.

(OR)

21.	Determine the dimensions of core and yoke for a 200 kVA, 50 Hz, single phase core type   transformer.  A cruciform core is used with distance between adjacent limbs equal to 1.6 times the width of core laminations.  Assume voltage per turn 14 V, maximum flux density 1.1wb/m2, window space factor 0.32, current density 3A/mm2 and stacking factor = 0.9.  The net iron area is 0.56d2 in a cruciform core where d is the diameter of circumscribing circle. The width of largest stamping is 0.85d.

22.	A 90 kW, 500 V, 50 Hz, 3 phase, 8 pole induction motor has a star connected stator winding accommodated in 63 slots with 6 conductors per slot.  If the slip ring voltage on open circuit is not to exceed 400 V, find a suitable rotor winding by estimating the number of slots, number of conductors per slot, coil span, slip ring voltage on open circuit, approximate full load current per phase in rotor.  Assume η =0.9 and power factor = 0.86.

(OR)

23.	Design a cage rotor for a 40 H.P, 3-phase, 400V, 50Hz, 6 pole, delta connected induction motor having a full load efficiency of 87% and a full load pf of 0.85.  Take D = 33 cm and L = 17 cm.  Stator slots = 54, conductors/slot = 14.  Assume suitably any missing data if any.

24.	Calculate the output co-efficient for a 1500 kVA, 2200 V, 3 phase, 10 pole 50 Hz, star connected alternator with sinusoidal flux distribution.  The winding has 60º phase spread, full pitch coils, ac = 30000 ampere conductors/m and Bav = 0.6 Wb/m2.  If the peripheral speed of the rotor must not exceed 100 m/sec and the ratio of pole pitch to core length is to be between 0.6 and 1, find D and L.  Assume an air gap length of 6 mm.  Find also the approximate number of stator conductors.

(OR)

25.	Estimate the stator core dimensions of the core, number of stator slots and number of stator conductors per slot for a 100kW, 3300 V, 50Hz, 12 pole, star connected slip ring induction motor.  Bav = 0.4 Wb/m2, ac = 25000 ampere conductors/m, an efficiency of 90%, power factor= 0.9.  Choose main dimensions to give best power factor.  The slot loading should not exceed 500 Ampere Conductor.