# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DIGITAL ELECTRONICS 								       Time: 3 hours

# Subject Code:	EE257		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Why is complementing a number representation needed?

2.	Convert (634)8 to binary.

3.	Define power dissipation.

4.	Define fan in.

5.	What do you mean by comparator?

6.	What is binary decoder?

7.	What are the classifications of sequential circuits?

8.	Define propagation delay.

9.	Define PLD.

10.	What are the features of VHDL?

PART – B (5 x 3 = 15 MARKS)

11.	Given the two binary numbers X = 1010100 and Y = 1000011, perform the subtraction

a.	X -Y and 	b.	Y - X using 2's complements.

12.	Design a CMOS logic gates for the function F= .

13.	Draw the logic diagram for two-input EX-OR gate using only NAND gates.

14.	What is the operation of SR flip-flop?

15.	What is programmable logic array? How does it differ from ROM?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Convert the following number from the given base to the other three bases listed in the 	table: (12)

Decimal		  Binary	            	 	Octal  	           Hexadecimal

369.3125    	   	?				?                              ?

?        	          10111101.101              	?                              ?

?                	     	?                              326.5			?

?                     	?	                        ?                             F3C7.A

b.	Prove the Demorgan’s theorem and draw the equivalent logic symbol 	(3)

(OR)

17.	a.	Design and implement the 3-bit Binary to Gray code conversion.             					(8)

b.	Write each of the following in its other cononical form using decimal notation.			(7)

i)	f(a,b,c) = ∑m (1,4,6)		ii)	f(a,b,c) = ∏M(2,5)

18.	a.	Recall that CMOS logic gates consist of a pull-down and a pull-up network comprising of nMOS 	and pMOS transistors respectively. Moreover, CMOS gates implement Inverting logic. Keeping 	this in mind, design a CMOS gate that implements the following Boolean expressions. Note 	that 	you are asked to design just one logic gate implementing the entire 	Boolean function. Use as 	few transistors as possible. Make sure to label the input (gate of each transistor) variables 	accordingly. Furthermore,  don’t forget to show where the output of the logic gate is.	        (5+5)

i)			ii)

b.	With help of input and output response of inverter to illustrate the propagation delay. 		(5)

(OR)

[P.T.O]

19.	a.	Draw the two input TTL Nand gate with totem-pole output and explain its operation.		(10)

b.	Draw the two input nMOS NOR logic circuits, Explain its operation.                   			(5)

20.	a.	Minimize the following function using the Quine-McClusky tabular method:

F(a,b,c,d) = ∑m(1,2,3,5,6,7,10,11)+d(9,12,15)                                                      			(10)

b.	Design the 1-bit magnitude comparator and explain its operation.						(5)

(OR)

21.	a.	Implement full subtractor using demultiplexer.										(7)

b.	Implement the following Boolean function using 8:1 multiplexer						(8)

22.	a.	An S-R latch constructed from NAND gates is shown in figure.   Determine the logic levels at 	points a, b and c under the following conditions:

i)	S = 0, R = 0 and Q = 0		ii)	As in (i), but S changes from 0→ 1

iii)	S = 0, R = 0 and Q = 1, and R changes from 0 → 1

Waveforms for S and R are shown in Figure (b). Draw the corresponding waveform for Q assuming that the initial value of Q =0.                                                    					(8)

b.	Design the 4-bit Johnson counter and explain its operation with help of timing diagram.	      	(7)

(OR)

23.	a.	Draw the circuit and explain a synchronous mod-12 counter using J-K flip-flops.			(12)

b.	How do synchronous counters differ from asynchronous counters?	        				(3)

24.	a.	Using basic gates, develop a VHDL code for a Fulladder.			        				(6)

b.	Develop a VHDL code for 4:1 mux using PROCESS statement.		        			(9)

(OR)

25.	a.	Implement the following functions using a PROM PLD

F1(a,b,c)= ∑m(2,3,4,6)

F2(a,b,c)= ∑m(0,1,5,7)

F3(a,b,c)= ∑m(4,5)

b.	Implement the following functions using 3x4x2 PLA

F1(a,b,c)= ∑m(1,2,3,6)

F2(a,b,c)= ∑m(0,1,3,6,7)