# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DIGITAL SIGNAL PROCESSING 						       Time: 3 hours

# Subject Code:	EE260		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define signal.

2.	What is impulse response?

3.	The number of stages for radix -2 DIF algorithm for N samples is ____________.

4.	The N-point IDFT of a sequence X(k) is ____________.

5.	How many numbers of addition and multiplications are required to realize a system H(z) having M zeros and N poles in direct form I realization.

6.	What is the necessary and sufficient condition for linear phase characteristics in FIR filter?

7.	Give the expression for signal to quantization noise ratio.

8.	What are the different quantization methods?

9.	What is pipeline depth?

10.	What are the factors that influence selection of digital signal processors?

PART – B (5 x 3 = 15 MARKS)

11.	Test whether the given signal is an energy signal or power signal? x(n)=u(n)-u(n-5).

12.	Compute the DFT of sequence (-1)n for N=4.

13.	Determine the order of digital Chebyshev filter that meets the following specifications:

1 dB ripple in the passband   and atleast 60 dB attenuation in stopband

14.	The filter co-efficient H= -0.673 is represented by sign magnitude fixed point arithmetic. If the word length is 6 bits, compute the quantization error due to truncation.

15.	What are the advantages and disadvantages of VLIW architecture?

PART – C (5 x 15 = 75 MARKS)

16.	a.	What is the input signal x(n) that will generate the output sequence 	y(n)={1,5,10,11,8,4,1} for a system with impulse response h(n)={1,2,1}.		         (5)

b.	Check if the following system linear, shift invariant and causal:

y(n)=3y2(n-1)-nx(n)+4x(n-1)-2x(n+1).											(10)

(OR)

17.	Determine the impulse response for the difference equation.

y(n)+3y(n-1)+2y(n-2)=2x(n)-x(n-1)

18.	Compute the 8-point DFT of the sequence

by using DIT –FFT algorithm.

(OR)

[P.T.O]

19.	a.	Compute a circular convolution of the two sequence x1(n)={1,2,2,1}and x2(n)={1,2,3,1} 	using concentric circle method.							         					(5)

b.	If X(k) is N point DFT of N point sequence x(n) then prove the following.

i)		ii) 		       (10)

20.	Design a digital butterworth filter for the following specification using impulse invariance method.

(OR)

21.	Design an Low Pass Filter using rectangular window if N=9 whose desired frequency response is

22.	Write short notes on

a.	Zero input limit cycle oscillations.						       						(10)

b.	Overflow limit cycle oscillations.							         				(5)

(OR)

23.	For the given transfer function H(z)=H1(z).H2(z) where  and. Find the output roundoff noise power. Calculate the value if b=3 (excluding sign bit).

24.	a.	Explain in detail about Harvard architecture.					       					(10)

b.	Write short notes on Replication.							        				(5)

(OR)

25.	Write short notes on

a.	Multiplier-accumulator.													      	(7)

b.	Pipelining 										          						(8)