# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MEASUREMENTS AND INSTRUMENTATION 			       Time: 3 hours

# Subject Code:	EE262		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	In a D’Arsonval galvanometer, why is an iron core used between the permanent magnet pole faces?

2.	A 1 mA ammeter has a resistance of 100 Ω.  What is the value of shunt resistance to be connected to convert it to a 1 A ammeter?

3.	Why is pressure coil made to move in the case of a dynamometer type wattmeter?

4.	What will be pointer position of power factor meter when it is disconnected from the supply?  Why?

5.	The sensitivity of wheatstone bridge is high when _________.

6.	Maxwell’s bridge is used for measurement of inductance of _________ Q coils.

7.	How are transducers classified (any two)?

8.	What is a thermocouple?

9.	What does a spectrum analyzer do?

10.	Why does an electronic voltmeter that use rectifiers employ negative feedback?

PART – B (5 x 3 = 15 MARKS)

11.	Derive an expression for torque in case of moving coil instruments and prove that it can be used for measurement of DC quantities only.

12.	A wattmeter has a current coil of 0.1 Ω resistance and a pressure coil of 6500 Ω resistance.  Calculate the percentage error if the wattmeter measures power in a 12A, 250V circuit with its current coil connected to the load side.

13.	The four arms of a Hay’s alternating current bridge are arranged as follows:  AB is a coil of unknown impedance, BC is a non – reactive resistor of 1000 Ω; CD is a non – reactive resistor of 833 Ω in series with a series capacitor of 0.38F, DA is non – reactive resistor of 16800 Ω.  If the supply frequency is 50Hz, determine the inductance and the resistance at the balanced condition.

14.	Brief the operation of a Resistive potentiometer.

15.	Draw the block diagram of dual channel oscilloscope.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Prove that dynamometer type moving coil instruments can be used to measure both AC 	and DC quantities from the first principles.						     				(10)

b.	A basic D’Arsonval meter movement with an internal resistance Rm = 100 Ω and a full scale current of Im = 1mA is to be converted into a multi – range d.c voltmeter with ranges of 0 – 10 V, 0 – 50V, 0 – 250 V and 0 – 500V.  Find the values of various resistances using the potential divider arrangement.							     					(5)

(OR)

[P.T.O]

17.	a.	Describe the construction and working principle of both types of moving iron instrument.

(10)

b.	How are Standards classified?  Explain each of them briefly			    			(5)

18.	a.	What are the reasons for inaccurate reading of wattmeter when used for measurement of 	power with low power factor?  How is it overcome?					   			(7)

b.	Brief the eight step procedure of calibrating the energy meter to measure the energy consumed with errors under all conditions.						  				(8)

(OR)

19.	a.	An energy meter is designed to make 100 revolutions of disc for one unit of energy.   	Calculate the number of revolutions made by it when connected to load carrying 40A at 	230V and 0.4 power factor for an hour.  If it actually makes 360 revolutions, find the 	percentage error.										  						(7)

b.	Explain how large power can be measured using nominal rating wattmeters in conjunction with instrument transformer.						  							(8)

20.	A 4 terminal resistor of approximately 50 resistance was measured by means of a Kelvin bridge having the following component resistances:

Standard resistor = 100.03; Inner ratio arms = 100.31 and 200; outer ratio arms100.24 and 200 ; resistance of link connecting the standard and the unknown resistance = 700.  Calculate the unknown resistance to the nearest 0.01 after deriving the necessary balance equation.

(OR)

21.	a.	With a neat phasor diagram obtain the balance equation of Anderson Bridge.		(10)

b.	Explain how earth resistance can be measured using earth tester.			 		(5)

22.	a.	Describe different modes of operation of piezo-electric transducers.  Derive expressions for 	voltage and charge sensitivities.												(10)

b.	A capacitive transducer uses two quartz diaphragms of area 750 mm2 separated by a distance of 3.55mm.  A pressure of 900 kN/m2 when applied to the top diaphragm produces a deflection of 0.6mm.  The capacitance is 370pF when no pressure is applied to the diaphragms.  Find the value of capacitance after the application of a pressure of 900kN/m2. 													   				(5)

(OR)

23.	a.	What is principle of operation of Hall Effect transducer?  What are its application?  	Explain 	any one application in detail.							  				(10)

b.	A thermocouple circuit gives an emf of 33.3V when measuring a temperature of 800°C with reference temperature of 0°C.  The resistance of the meter coil, Rm is 50Ω and a current of 0.1mA gives full scale deflection.  The resistance of junctions and leads, Re is 12Ω.  Calculate i) Resistance of the series resistance if the temperature of 800° is to give full scale deflection ii) the approximate error due to rise in 1 Ω in Re.				(5)

24.	a.	Explain the operation of ramp type digital voltmeter with a neat block diagram.		(10)

b.	Mention any five applications of X – Y recorders                                         	  		(5)

(OR)

25.	a.	With a neat block diagram explain the operation of digital multimeter.				(7)

b.	Define the critical pulse parameters of a pulse generator with illustration.   	  		(8)