# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTRICAL ENGINEERING 							       Time: 3 hours

# Subject Code:	EE296		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Name the three basic types of circuit elements.

2.	If the phase angle θ between voltage and current is 45°, what is the power factor?

3.	What is the limitation of superposition theorem?

4.	What is the phase angle of a series RLC circuit at resonance?

5.	Write any one application of a DC series motor.

6.	What is the relation between synchronous speed, supply frequency and number of poles of a three-phase induction motor?

7.	Write the unit of Luminous flux.

8.	The colour of light depends on _________ and __________.

9.	Why should a voltmeter have a high electrical resistance?

10.	 Write any one advantage of concealed wiring system.

PART – B (5 x 3 = 15 MARKS)

11.	Given v = 20sin (ωt + 30°) and i = 18sin(ωt - 40°), determine the phase relationships and sketch the waveform.

12.	Define quality factor Q of the coil.

13.	Why is iron core used in a transformer?

14.	State the laws of illumination.

15.	What is UPS?

PART – C (5 x 15 = 75 MARKS)

16.	For the circuit shown in figure, determine the following

a.	Total resistance RT. 					b.	Total current in the circuit IT.

c.	Power delivered by the voltage source	d.	Current passing through each resistor.

e.	Power dissipated by the resistors.

(OR)

17.	Write the mesh equations for the circuit shown in Figure. Determine the load current and the power delivered to load.

15

[P.T.O]

18.	Determine the Thevenin’s equivalent circuit across AB for the circuit shown in Figure. Using Thevenin’s equivalent circuit find the current in 3Ω resistor.

B

(OR)

19.	Find the values of L at which the circuit resonates at a frequency of 1000 rad/sec in the circuit shown in Figure.

20.	Describe the construction and working principle of three-phase induction motor.

(OR)

21.	Explain the construction and working of variable reluctance stepper motor.  Write the applications of stepper motors.

22.	Describe with a neat diagram the construction and working of a sodium vapour lamp. Mention its applications.

(OR)

23.	a.	Draw the schematic diagram of seven-segment display using LEDs and explain its 	function. Write the advantages of LED. 											(10)

b.	Write a short note about LCD display.  											(5)

24.	a.	Draw the 2-way lighting circuit diagram and explain how a single lamp is controlled by 	two 2-way switches (staircase lighting)    										(10)

b.	With the help of circuit diagram, explain the measurement of power in a DC circuit using voltmeter and ammeter.  														(5)

(OR)

25.	What is the function of the choke and a starter in fluorescent tube lighting?  Explain the working of a fluorescent tube with the help of the circuit diagram.