# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	SIMULATION OF POWER ELECTRONICS SYSTEMS	 Time: 3 hours

# Subject Code:	EE335			         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Write short notes on need for simulation and also brief the limitations of PSPICE.	(8)

b.	What are the types of circuit analysis available in PSPICE and describe the entire dot 	commands required for analyzing the circuits.					           			(12)

2.	a.	Explain the static and dynamic model of a p-n junction diode and write the Pspice circuit 	file for a half bridge rectifier circuit using .MODEL command.                         		(15)

b.	Write the required PSPICE statements to generate the following source waveforms in figure.																		(5)

(OR)

(OR)

3.	a.	For the given PSpice schematics in figure generate the PSpice circuit file to plot the 	Fourier components of load current up to 50th order. The switch S1 parameters are 	(RON=1mΩ, ROFF=10E+9, VON=25mV, VOFF=0.0)          		                     		(15)

b.	Write short notes on the following Dot commands of PSPICE with suitable example.	(5)

i)	PARAM	    		ii)	SUBCKT

4.	a.	Write short notes on line space function transpose operator and colon operator in 	MATLAB.																	(5)

b.	Write the output of the following MATLAB coding.								(5)

x = [1 2 3; 4 5 6; 7 8 9];

r = x(:,2);

c = x(1,:);

x

r

c

[P.T.O]

c.	Briefly discuss to convert S-functions to blocks in MATLAB SIMULINK environment.

(10)

(OR)

5.	a.	With the help of suitable power electronic applications explain the for loop and m file-	Function in MATLAB. 								                     				(10)

b.	Briefly discuss the given two dimensional graphics functions in MATLAB.	          	(10)

i)	Plot		ii)	Subplot		iii)	Label		iv)	Semilogy	v)	Polar.

6.	a.	Draw the schematic symbol and explain the functions of proportional, integral, 	differential and PI controllers in PSIM environment and give the suitable example of 	these converters in power electronics circuit.					                      		(10)

b.	Briefly discuss the different types of sources in PSIM environment.		           		(10)

(OR)

7.	a.	With the suitable example explain the wave form processing in PSIM.	           		(10)

b.	Write short notes on the following blocks in PSIM.				           			(10)

i)	abc to dqo transformation		ii)	Lookup table

iii)	Control power interface block.

8.	a.	Write short notes on repeat sequence block in MATLAB SIMULINK. And also 	comment the following statements.

*	Sine wave can be generated using repeat sequence block.

*	Saw tooth wave cannot be generated using repeat sequence					(4)

b.	Draw the MATLAB SIMULINK schematics of a three phase diode bridge rectifier circuit with R load. Briefly discuss the required MATLAB blocks to analyze the following parameters. 										           						(16)

*	The THD spectrum of source current.

*	rms, average value of the load, source currents.

*	Third order harmonic component of the source current.

*	Ripple factor and power factor.

(OR)

9.	a.	With suitable example explain the following MATLAB SIMULINK toolboxes. 		(10)

i)	Breaker.		ii)	Universal Bridge.			Iii)	Total Harmonic Distortion.

iv)	Fourier. 		v)	To work space.

b.	What are the advantages of the state space representation? Derive the state space model of the given Dc to DC power electronic converter shown in figure.	       		          	(10)