# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	BASIC ELECTRONICS AND INSTRUMENTATION		       Time: 3 hours

# Subject Code:	09EI101		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is a PN junction diode?

2.	What are different types of FET?

3.	What are universal gates?

4.	What is Boolean algebra?

5.	Write the principle function of primary standards.

6.	What is meant by calibration?

7.	Define transducer.

8.	Define pH of a liquid.

9.	Write the significance of multiplier in DC voltmeter.

10.	What are the basic components of a digital instrument?

PART – B (5 x 3 = 15 MARKS)

11.	Draw the VI characteristics of a thyristor and explain.

12.	Construct the RS flip flop using NOR gate and explain through truth table.

13.	How are deflecting instruments classified? Give example.

14.	Describe the basic principle of translational velocity transducers.

15.	Draw the basic block diagram of a digital frequency meter and outline the significance of each block.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Draw the VI characteristics of Zener diode.  Explain how a Zener diode can be used as 	voltage regulator circuit.					     									(7)

b.	Draw the circuit of a centre tapped full wave rectifier and explain its operation with relevant waveform.							    								(8)

(OR)

17.	Explain the working of NPN transistor. Draw the CE configuration of NPN transistor and explain with input and output characteristics curves.

18.	Explain the operation of half adder and full adder with logic circuit and truth table.

(OR)

19.	What are counters? Explain the operation of a four bit ripple counter with relevant circuit diagram and truth table. Also discuss its applications.

20.	Explain the static and dynamic characteristics of an instrument.

(OR)

21.	What are the possible errors encountered in an instrument? Explain the causes of errors and remedial measures to be done to reduce the errors.

[P.T.O]

22.	Describe the principles, construction and working of magnetic flow meters. Also discuss their advantages and disadvantages.

(OR)

23.	Explain the principles, construction and working of LVDT for displacement measurement.  Discuss its characteristics, advantages and disadvantages.

24.	a.	Explain the basic operation of DC ammeter and discuss how the range of measurement can 	be extended.						    											(7)

b.	Draw the diagram of a Multimeter and explain.				    					(8)

(OR)

25.	Discuss with a neat diagram, a method of realizing a seven segment numeric display using LEDs.