# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTRONIC CIRCUITS 								       Time: 3 hours

# Subject Code:	09EI221		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define rectifier.

2.	Ripple factor is ___________ proportional to load resistance in the inductor filter and __________ proportional to load resistance in the capacitor filter.

3.	Define rise time in transistor switching.

4.	What is a clipper?

5.	A circuit that increases the amplitude of the given input signal is ________.

6.	Define CMRR.

7.	What is Class A amplifier?

8.	Mention the types of negative feedback connection.

9.	_________ type of crystal is used as a resonant circuit in crystal oscillator.

10.	Mention the types of sine wave oscillator.

PART – B (5 x 3 = 15 MARKS)

11.	Define the following terms with reference to rectifiers: 	a) Ripple factor 	b)	Transformer Utilization Factor 	c)	 Peak Inverse Voltage.

12.	How can a high pass RC circuit be used as a differentiator?

13.	List the important characteristics of common base BJT amplifier.

14.	A voltage series negative feedback amplifier has a voltage gain without feedback of A = 500, input resistance Ri = 3 k, output resistance Ro=20 k and feedback ratio =0.01. Calculate the voltage gain, input and output resistance of the amplifier with feedback.

15.	In a Hartley oscillator, the value of the capacitor in the tuned circuit is 500 pF and the two sections of the coil have inductances 38 H and 12 H. Find the frequency of oscillations and the feedback factor .

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the expressions for average and rms value of voltage of a full wave rectifier with 	resistive load and hence deduce ripple factor, efficiency and transformer utility factor. 	Sketch necessary waveforms.                    										(9)

b.	A 230 V, 60 Hz voltage is applied to the primary of a 5:1 step down transformer used in a full wave rectifier having a load of 900 ohm. If the diode resistance and secondary coil resistance together has a resistance of 100 ohm, determine (i) d.c voltage across load (ii) d.c current flowing through the load (iii) d.c. power delivered to load (iv) PIV across each diode (v) ripple voltage. 														(6)

(OR)

17.	Draw the block diagram of a switched mode power supply and explain its operation with necessary waveforms. Mention its advantages.

18.	Explain in detail the square wave response of high pass RL circuit and low pass RL circuit.

(OR)

[P.T.O]

19.	Describe the operation of the following with neat circuit diagrams:

a.	Positive and negative clipper circuits 		b.	Clamper circuit.

20.	Draw the small signal equivalent circuit of FET amplifier in Common Source and Common Drain configuration and derive the necessary equations for voltage gain, input impedance and output impedance.

(OR)

21.	a.	A voltage source of internal resistance Rs =1200 ohm drives a CB transistor amplifier 	having a load resistance of RL =1000 ohm. The h-parameters are hib =22 ohm, hrb =3x10-4, 	hfb =-0.98 and hob =0.5 A/V. Compute the current gain, voltage gain, input impedance 	and output impedance. 														(7)

b.	Write brief notes on Cascade amplifier. 											(8)

22.	With a neat sketch explain the working of a Class B Push Pull amplifier and derive the expression for its efficiency.

(OR)

23.	Draw the block diagram of current series feedback circuit and explain its operation. Derive the expressions for feedback ratio, voltage gain, input and output resistance.

24.	a.	Draw the circuit diagram of Wien-Bridge oscillator and explain its working principle and 	operation.  					           											(11)

b.	If the RC network in a Wein bridge oscillator consists of resistors of 200 k and capacitors of 300 pF, calculate the frequency of oscillations.									(4)

(OR)

25.	With a neat diagram, explain the operation of a Schmitt Trigger circuit. Mention few of its applications.