# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination –  April/May 2011

# Subject Title:	DIGITAL SYSTEM DESIGN 							 Time: 3 hours

# Subject Code:	10EI307			         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain with neat diagram the Xilinx 3000-Logic Cell Array, Configurable Logic Block.

(12)

b.	List out the differences between the Semi-custom and Full-custom layout design styles.

(8)

2.	a.	What are the elements used in ASM charts? Discuss in detail.						(10)

b.	Design a combinational using a PROM. The circuit accepts 3-bit binary number and generates its equivalent Excess-3 code.											(10)

(OR)

3.	a.	A combinational circuit is defined by the functions:

f1 = ∑m (3,5,7)				f2 = ∑m (4,5,7)

Implement the circuit with a PLA having 3 inputs, 3 product terms and two outputs.  (10)

b.	Draw the basic structure of PAL and explain its working.			     			       (10)

4.	a.	List out the basic design steps for the design of a clocked synchronous sequential circuit.

(10)

b.	Discuss how excitation tables, state tables and state diagrams are used to analyse a synchronous sequential network.												(10)

(OR)

5.	Design a clocked sequential machine using T flip-flops for the following state diagram. Use state reduction if possible. Also use straight binary state assignment.

0/0

1/1

1/0

0/0

0/1

1/0

1/0

0/1

a

b

c

d

6.	a.	Write the VHDL code for a given full adder circuit using structural method. 		(10)

SUM

CARRY

W  SUM

W_ CARRY2

W_ CARRY1

Module1

Module2

Module3

CARRY_IN

A

B

Full adder: 2 half adders + 1OR-gate

b.	Develop a VHDL code for 4:1 Mux using PROCESS statement.	                   		(10)

(OR)

[P.T.O]

7.	a.	Develop a VHDL code for synchronous 4-bit up/down counter with load enable.		(10)

b.	Explain any FIVE types of sequential statements along with syntax and example for each.                                                                                   						      (10)

8.	a.	A demultiplexer passes a single input to one of many output lines, depending on a 	control input. Write a Verilog description for a 1-to-2 demultiplexer using only primitive 	logic elements.						             								      (10)

in

s

z0

z1

| S | z0 | z1 |
| --- | --- | --- |
| 0 | in | 0 |
| 1 | 0 | in |

b.	Write a Verilog description for an 8-bit decoder called addr_dec8 using only the 1-to-2 demultiplexer that you wrote in the previous problem (8a).  					(10)

Out[7]

Out[6]

Out[5]

Out[4]

Out[3]

Out[2]

Out[1]

Out[0]

in [2-0]

| in[2) | in[1] | in[0] | out[7] | out[6] | out[5] | out[4] | out[3] | out[2] | out[1] | out[0] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

(OR)

9.	a.	Explain the following with a suitable example for each						(10)

i)	Delay based timing		ii)	Event based timing

b.	Develop a verilog code that perform logical shift left and logical shift right.		(10)