# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	CONTROL SYSTEMS 									       Time: 3 hours

# Subject Code:	EI203		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is disturbance in a system?

2.	What is the function of synchro control transformer?

3.	What is the effect of damping factor in overshoot of the response?

4.	What is steady state error of type 1 system due to step input?

5.	Define absolute stability.

6.	State the magnitude criterion of the root locus.

7.	What is the frequency response of a system?

8.	Define gain margin.

9.	Why are derivative controllers not used alone in a system?

10.	What is fuzzification?

PART – B (5 x 3 = 15 MARKS)

11.	What is synchro transmitter and synchro receiver?

12.	Define transient response and steady state response.

13.	What is breakaway and breakin point?

14.	Sketch the approximate magnitude Bode plot for G(s) = 1/(1+sT).

15.	State the essential characteristics of fuzzy logic.

PART – C (5 x 15 = 75 MARKS)

16.	Obtain the transfer function of the block diagram below using block diagram reduction technique.

G1

G2

H1

C(s)

R(s)

G3

H2

H3

(OR)

17.	Derive the mathematical model and transfer function of field controlled separately excited DC motor.

[P.T.O]

18.	For a system shown below, assume T= 5 sec and J=1500 kg m2 to determine the following:

A(Ts+1)

2

Js2

R(s)

C(s)

a.	The value of A so that overshoot will be 1.9%.

b.	Kv and steady state error for input r(t)= t.

(OR)

19.	For the system with loop transfer function, determine the type of system, error coefficient, and the error for the following inputs.

a.	r(t) = 6				b.	r(t) = 10+4t+1.5t2

20.	Sketch the root locus for the system.

(OR)

21.	The system shown in figure below has the oscillation of 2.5 rad/sec. Determine the value of K for marginal stability and p.  There are no poles in right half of the plane.

1

s3+ps2+3s+2

C(s)

R(s)

K(s+2)

22.	For , draw the Nyquist plot and determine the range of k for stability.

(OR)

23.	A unity feedback system open loop transfer function   Draw the Bode plot.

24.	Explain the temperature control of a system with fuzzy logic.

(OR)

25.	Write a procedure to design a PI controller.