# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DIGITAL CONTROL SYSTEMS 						       Time: 3 hours

# Subject Code:	EI214		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	State Shannon’s sampling theorem.

2.	Define droop rate with respect to sample and hold operation.

3.	What is the stable region of Z-plane?

4.	What is bilinear transformation?

5.	Give the state model of SISO, second order system.

6.	Give any one advantage of parallel realization.

7.	State the condition when a controller is said to be physically unrealizable.

8.	What is ringing in digital controller?

9.	In a 3-phase stepper motor number of rotor teeth is 8, what is the step angle?

10.	Give any two advantages of microprocessor based control systems.

PART – B (5 x 3 = 15 MARKS)

11.	Obtain the inverse Z-transformation of the function   G(z) = .

12.	Determine the stability of the sampled data system whose open loop transfer function is given by   A(z) = .

13.	Obtain the solution for discrete state equations using z-transform.

14.	Brief about the design specifications of Kalman’s algorithm.

15.	Draw the Chopper drive circuit and mention its merits and demerits.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the transfer function of Zero Order Hold.									(8)

b.	Briefly explain the operation of R-2R ladder 3-bit D/A converter with neat circuit.	(7)

(OR)

17.	a.	Explain the principle and operation of Successive Approximation type A/D Converter with 	neat diagram.																(8)

b.	Describe in detail about the issues in selecting the sampling frequency.				(7)

ZOH

T

T=1 Sec

Y(s)

R(s)

18.	For the sampled data system given below, find the response y(k) for unit step change in input r(k)

(OR)								       [P.T.O]

19.	a.	Determine the stability using Jury’s test	Δ(Z) = Z4- 1.2Z3+0.07Z2+0.3Z-0.08		(8)

b.	Obtain the steady state errors Kp*, Kv* and Ka* for given G(z) = 	(7)

20.	a.	Find state transition matrix Ф(k) if the system state matrix is given as A= (7)

b.	Obtain the parallel realization for the given pulse transfer function

(8)

(OR)

21.	Obtain the phase variable form of state model of the following system and find the controllability of the same.  y(k+3) + 4y(k+2) +  9y(k+1) + 6y(k) =u(k)

22.	a.	Derive the digital equivalent of position form of PID algorithm.					(8)

b.	Explain any digital control system using microcontroller with block diagram.		(7)

(OR)

23.	Design Deadbeat controller algorithm for a system having a model Gp(s)=e-3s18s+1	with unity feedback.  The sampling period T=1.2 sec. Also check whether the controller is physically realizable or not.

24.	With relevant diagrams, explain the microprocessor based temperature control system.

(OR)

25.	Explain the interfacing of stepper motor to microprocessor with the help of neat circuit diagram.