# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	INSTRUMENTATION AND CONTROL IN PETROCHEMICAL

# INDUSTRIES 								      	   		       Time: 3 hours

# Subject Code:	EI225		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	________ separates a mixture on the basis of a difference in the composition of a liquid and that of the vapor formed from the liquid.

2.	In________ distillation the feed concentration is relatively constant, while in ________ distillation it is rich in light components at the beginning and lean in light components at the end.

3.	What is adiabatic and nonadiabatic dryer?

4.	________designates a batch dryer that operates at close to atmospheric pressure and obtains the heat drying from the hot air supply.

5.	Degree of Freedom = (Number of ________) – (Number of ________).

6.	Draw the symbol of steam heater.

7.	What are the methods used to measure the product density?

8.	Draw the schematic diagram of single effect evaporator.

9.	Mention the reliable control methods of measurements commonly used in water treatment.

10.	Commonly used reductants are ________

PART – B (5 x 3 = 15 MARKS)

11.	What is the purpose of fractionator?

12.	What is the function of tray and truck dryer?

13.	Give a note on the degree of freedom in liquid to liquid heat exchangers and steam heated reboilers.

14.	Explain the multiple effect evaporation.

15.	What is meant by chlorine demand and residual chlorine?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Write a short note on Reboilers design.												(8)

b.	Explain the working of batch distillation with neat diagram.							(7)

(OR)

17.	With a neat sketch explain the operation of

a.	Column pressure control with inerts present											(7)

b.	Column pressure control with vapor distillate and inerts present.						(8)

18.	Explain the principle and working of the advanced batch dryer control system with neat sketch.

(OR)

19.	Discuss about the operation of rotary dryers.

20.	Explain with neat diagram the operation of steam heater under feedforward control.

(OR)

21.	Explain the methods by which the condenser control is achieved.

[P.T.O]

22.	With neat sketch explain any 3 types of evaporators.

(OR)

23.	Explain the feedback control system and cascade control system for evaporators with necessary 	diagrams.

24.	a.	Explain the operation of continuous oxidation of cyanide waste with chlorine.		 	(8)

b.	Give a note on calcium carbonate precipitation control system.							(7)

(OR)

25.	a.	Briefly explain the chemical reduction process of chromium waste with sulfur dioxide.	(8)

b.	Draw the schematic diagram of acid waste neutralization systems.						(7)