# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTRONIC CIRCUITS 								       Time: 3 hours

# Subject Code:	EI238		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define ripple factor of an ac waveform.

2.	Calculate the average value of half wave rectifier having frequency 100Hz.

3.	Draw the 	a) Impulse	and 	b) step functions.

4.	Differentiate between attenuator and amplifier.

5.	Draw the characteristics (gain Vs frequency) of RC coupled amplifier.

6.	Two transistors having current gain 1 and 2 are connected in Darlington connection.  Find 	the overall current gain.

7.	An amplifier has open loop gain of 10,0000.  Find the gain in dB.

8.	Sate the theoretical maximum efficiency of class A power amplifier.

9.	What are advantages of crystal oscillator?

10.	Draw the circuit of Astable multivibrator.

PART – B (5 x 3 = 15 MARKS)

11.	Find the average value of full wave rectifier output waveform (Assume frequency = 750Hz).

12.	For the circuit of figure.  Assume the diodes are ideal. Draw the o/p waveform.

R

Vs

vo

vi

vi = Vm sin t

13.	Define CMRR.

14.	Draw the circuit of PUSH-PULL complementary symmetry power amplifier.

15.	Calculate the frequency of oscillation in rad/sec of RC oscillator having R = 1k and C=0.1F.

PART – C (5 x 15 = 75 MARKS)

16.	Draw the circuit of full wave rectifier and explain the working principle with relevant diagram.

(OR)

17.	Write short notes on

a.	Filters used in Regulated power supply 			b.	SMPS

18.	Explain the working principle of differentiator and design such circuit for 1kHz square wave.

(OR)

19.	Describe any one application of 	a.	clipping circuit		b.	Clamping circuit.

20.	Derive the voltage gain formula for single stage common drain amplifier (at mid frequency).

(OR)

21.	Explain the RC coupled amplifier circuit and its frequency response.

[P.T.O]

22.	Derive the voltage gain with feedback and input resistance of voltage series feedback circuit.

(OR)

23.	Describe crossover distortion and the method to eliminate the same in power amplifier.

24.	Derive the frequency of oscillation for Hartley oscillator.

(OR)

25.	Derive the frequency of Astable multivibrator using BJT in terms of circuit components.