# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	SENSORS AND TRANSDUCERS 						       Time: 3 hours

# Subject Code:	EI247		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Explain hysteresis error.

2.	Differentiate between Reproducibility and Repeatability.

3.	Write the basic classification of transducers.

4.	Define Transducer.

5.	Discuss the merits of thermocouple.

6.	RTD is a

a.	Zero order 	b.	I order system 	c.	II order system 		d.	No order system

7.	Explain the principle of capacitance transducer.

8.	Give the application of piezoelectric transducer.

9.	Discuss the principle of optical transducer.

10.	List the application of pH sensor.

PART – B (5 x 3 = 15 MARKS)

11.	Derive the response of a first order system having a gain k and a time constant Γ for a unit step change input.

12.	Write the basic principle of piezoelectric transducer.

13.	Compare RTD with Thermistor.

14.	Write the principle and construction details of Hall Effect transducer.

15.	Explain passive transducer.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Write short notes on the static characteristics of the instrument.						(7)

b.	Explain the general block diagram of a measurement system with example. 				(8)

(OR)

17.	The table below gives experimental data

| Temperature | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of occurrence | 1 | 2 | 3 | 6 | 7 | 6 | 4 | 2 | 1 |

Calculate a) arithmetic mean (b) mean deviation (c) standard deviation (d) probable error of one reading e) Standard deviation and the probable error of the mean (f) Standard deviation of the Standard deviation.

18.	Describe the construction and working principle of RTD. Describe the advantages of 3 wire and 4 wire constructions.    		 	         									(9+3+3)

(OR)

[P.T.O]

19.	a.	A strain gauge is bonded to a beam of 0.1 mm long and has a cross sectional area 4 cm2 	E=207 GN / M2. The strain gauge has an unstrained resistance of 240 Ω and a gauge factor 	of 2.2. When load is applied, the resistance of gauge changes by 0.013 Ω. Calculate the 	change in the length of the steel beam and the amount of force applied to the beam.		(7)

b.	Explain synchros as an error detector.					       							(8)

20.	a.	A capacitive transducer circuit is used for measurement of linear displacement. The transducer 	is a parallel plate air capacitor wherein the capacitance can be changed by changing the 	distance between the plates. This transducer is to be used for dynamic measurements. 	Suppose a flat frequency response with an amplitude ratio within 5% is required down to a 	frequency range of 30 Hz, what is the minimum allowable value of time constant? Calculate 	the phase shift at this frequency. The area of plates is 300 mm2 and the distance between 	plates is 0.25 mm. Calculate the value of series resistance R. What is the amplitude ratio at 	5Hz with the above time constant?	                					     					(10)

b.	Explain the application of capacitive transducer in Level measurement. 					(5)

(OR)

21.	a.	A Piezo-electric transducer has a capacitance of 1000 pF and a charge-sensitivity of 40x10-3 	C/m. The connecting cable has a capacitance of 300 pF while the oscilloscope used for 	readout has a input resistance of 1 MΩ with a parallel capacitance of 50 pF.				(10)

i)	What is the sensitivity (V/m) of the transducer alone?

ii)	What is the high frequency sensitivity (V/m) of the entire measuring system?

iii)	What is the lowest frequency that can be measured with 5% amplitude error by the entire 			system?

iv)	What is the value of an external shunt capacitance that can be connected in order to extend the range of 5 % error down to 10 Hz?

b.	Explain the frequency response of piezoelectric transducer. 		      					(5)

22.	a.	Explain the basic principle of a hall sensor and show how it can be used for sensing 	displacement using necessary diagram					       							(7)

b.	The hall-effect transducer is used for the measurement of a magnetic field of 0.5Wb/m2. The 2mm thick slab is made up of bismuth for which the hall’s coefficient is -1 X 10-6 {V m/A-wb m-2} and current of 3A. Find hall voltage.               									(8)

(OR)

23.	Explain the principle and constructional details of pH sensor.

24.	a.	Differentiate between active and passive transducers.				    					(10)

b.	Explain inverse transducer.						     								(5)

(OR)

25.	a.	Explain the various classifications of Transducer.  			    						(10)

b.	Explain the various factors influencing for choosing the transducer.              				(5)