# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	COMMUNICATION ENGINEERING 					       Time: 3 hours

# Subject Code:	EI248		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Why are carrier frequencies generally selected in high frequency range than low frequency range?

2.	Define the term modulation index for amplitude modulation.

3.	Write the frequency range of commercial FM radio transmission.

4.	Define noise factor.

5.	State sampling theorem.

6.	What is the major limitation of PCM?

7.	Name a fiber optic source and detector.

8.	State any two advantages of coaxial cable over twisted cable.

9.	Name the modulation technique used for audio and video signal transmission in television.

10.	Define frequency reuse in cellular communication.

PART – B (5 x 3 = 15 MARKS)

11.	What are the advantage and disadvantages of SSB over DSBFC?

12.	What are the advantages and disadvantages of FM over AM?

13.	What is DPCM? Write its advantages.

14.	Distinguish between TDM and FDM.

15.	What is modem? Give the classification of modems.

PART – C (5 x 15 = 75 MARKS)

16.	Describe the method of generating an amplitude modulated signal and derive expression for its frequency spectrum and bandwidth.

(OR)

17.	Draw the waveforms for frequency modulation and derive an expression for the frequency modulated signal.

18.	Draw the block diagram of a superheterodyne receiver and explain the operation.  Write frequency components present at the output of each block if audio frequency is 1 kHz and carrier frequency is 540 kHz.

(OR)

19.	Write notes on the following:													      (3 x 5)

a.	Thermal noise		b.	Shot noise		c.	Avalanche noise

20.	What is delta modulation? With neat diagrams and waveforms, explain the transmitter and receiver operation for delta modulation.

(OR)

[P.T.O]

21.	Describe FSK with necessary equations.  With neat diagrams, explain the operation of FSK modulator and demodulator.

22.	What is error correction?  Explain any two methods of error correction techniques.

(OR)

23.	Explain the successive approximation method of analog to digital converter with a neat block diagram.

24.	With a neat block diagram, explain the working of a colour television receiver.

(OR)

25.	Draw the basic block diagram of a satellite communication system and explain its operation.