# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	INDUSTRIAL INSTRUMENTATION 					       Time: 3 hours

# Subject Code:	EI256		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What are the materials used for the manufacturing of Bourdon Tubes?

2.	Name two commonly known thermal conductivity pressure gauges.

3.	Specify the different types of orifice.

4.	List the different units of flow.

5.	What is the resistance of RTD made up of Platinum at 0oC?

6.	Name the principle of operation of thermocouple.

7.	What is the advantage of Air purge system?

8.	Define swelling.

9.	What is the signal transmitted from HART?

10.	Which software plays an important role in Virtual Instrumentation?

PART – B (5 x 3 = 15 MARKS)

11.	What is Manometer? List its different types.

12.	What is Rotameter? What are the disadvantages of Glass Rotameter?

13.	What are Thermopiles?  Where is it used?

14.	How is solid level measured in a Tank?

15.	Differentiate Virtual Instrumentation from Traditional Instrumentation.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the different types of Bourdon gauges.

(OR)

17.	Explain the measurement of Pressure using Strain gauge.

18.	Explain Rotameter type variable area flow meters with its advantages and disadvantages.

(OR)

19.	Describe with neat sketch two types of Ultrasonic Flow meters.

20.	What is meant by Cold Junction compensation? What is the practical importance of it? Explain how it is obtained?

(OR)

21.	a.	Explain Optical Pyrometer.													(7)

b.	Explain the working of Four lead Resistance temperature detectors.					(8)

22.	a.	Explain the principle, working of Air purge level measurement system with its advantages.

(10)

b.	Explain the Non- conducting type optical level sensor.							(5)

(OR)

[P.T.O]

23.	Explain the construction and working of level measurement using Displacer devices.

24.	What is virtual Instrumentation? Describe different components of Virtual Instrumentation.

(OR)

25.	a.	Explain PC based Data Acquisition system.										(8)

b.	Explain in detail about HART Protocol.											(7)