# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ELECTRONIC INSTRUMENTATION 					       Time: 3 hours

# Subject Code:	EI261		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Voltmeters are connected in _________ with the circuit whose voltage is to be measured.

2.	What are the two types of spectrum analysis?

3.	What is the source for emission of electrons in CRT?

4.	What is the need of vertical amplifier in a CRO?

5.	What is the normally employed frequency divider circuit in a digital counter?

6.	Give the application of Q meter?

7.	What are the classifications of Digital displacement transducer?

8.	What are the types of Velocity measurements?

9.	What is the illumination systems used in seven segment displays?

10.	List the typical applications of UV recorders.

PART – B (5 x 3 = 15 MARKS)

11.	What is vector impedance meter?

12.	What is the purpose of Horizontal amplifier in a CRO?

13.	Draw the structure of successive approximation type ADC.

14.	Why is Frequency measurement necessary? What are the types of Frequency measurement?

15.	What are 7 segment displays?

PART – C (5 x 15 = 75 MARKS)

16.	Explain with neat sketch Permanent magnet moving coil instrument used for d. c. measurements.

(OR)

17.	What is Spectrum analyzer? Explain the spectrum analyzer with its block diagram and waveforms.

18.	Explain the working principle of time base generator.

(OR)

19.	Draw the structure of Cathode ray tube and describe its different parts.

20.	Explain any two types of Digital Voltmeters.

(OR)

21.	Explain with neat diagram the working of Digital storage oscilloscope.

22.	Explain the digital methods of measuring angular velocity.

(OR)

23.	Explain the Optical digital displacement transducer with necessary sketches.

[P.T.O]

24.	What is X-Y recorder? Explain with suitable diagram, the working of X-Y recorder. Describe its applications.

(OR)

25.	a.	Explain the working of ultra violet Recorder.									(10)

b.	List the advantages of Magnetic tape recorders.									(5)