# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DIGITAL SIGNAL PROCESSING 						       Time: 3 hours

# Subject Code:	EI262		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	An LTI system has impulse response h[n]. The response of the system to an input x[n] can be determined by periodic convolution of x[n] and h[n]. Say True/False.

2.	Name the type of convolution that eliminates the delay in response due to long input sequences.

3.	In FIR filter design, a sharp cut off frequency can be achieved by __________  the main lobe width of the window spectrum.

4.	An LTI system whose frequency response acts as a spectral shaping function to the different frequency components in the input signal is called ___________.

5.	Trapezoidal numerical integration approach to the differential equation represented by H(s) to derive H(z) is used in _____________ technique in the design of an IIR filter.

6.	The LTI system represented by the difference equation

y[n] = x[n]+2x[n-1]-3x[n-2]+y[n-1]+3y[n-2]  is a ______(IIR/FIR) system.

7.	Draw the schematic of Harvard architecture.

8.	Expand VLIW.

9.	List the On-chip peripherals in TMS processor.

10.	What is the use of NORM instruction in TMS processor?

PART – B (5 x 3 = 15 MARKS)

11.	State and prove the periodicity property of DFT.

12.	Realize the LTI system whose impulse response is given by h[n]={0.06, 0.01, -0.05, -0.25, 0.61, -0.25, -0.05, 0.01, 0.06}using minimum number of multipliers.

13.	Use impulse invariant transformation to determine H(z) for the analog transfer function H(s)=1s+5. Assume T=1 second.

14.	List the relative merits and demerits of RISC and CISC processors.

15.	What is Bit reversed addressing?

PART – C (5 x 15 = 75 MARKS)

16.	Compute the 8 point DFT of the sequence x[n] by radix 2 DIT – FFT algorithm.

x[n]={ 1, 2, 3, 1, 3, 2, 2, 1}

(OR)

17.	Find the system function and impulse response of the LTI system described by the difference equation y[n] - 3y[n-1] - 4y[n-2] = x[n] + 2x[n-1].

18.	Using a Hamming window, design a high pass filter with pass band gain of unity, cutoff frequency of 1000Hz and working at a sampling frequency of 5 kHz. The length of the impulse response should be 9.

(OR)

19.	Design a lowpass FIR filter using frequency sampling technique having cutoff frequency of radians/s. The filter should have linear phase and length of 17.

[P.T.O]

20.	Design a digital Butterworth filter satisfying the following requirements using bilinear transformation technique.

.

(OR)

21.	Obtain the direct form I, direct form II and Cascade realization of the LTI system

H(z) =.

22.	What is instruction pipelining? Explain with an example how pipelining increases the throughput efficiency.

(OR)

23.	Bring out the significance of MAC and MACD operations in a Digital Signal Processor with an example.

24.	Draw the internal architecture of TMS processor and explain the function of each block.

(OR)

25.	What are the special Addressing modes in a TMS processor? Explain each one with an example.