# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DIGITAL SIGNAL PROCESSING 						       Time: 3 hours

# Subject Code:	10MT203	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	When a discrete time signal is called periodic?

2.	Define time invariant system.

3.	What is zero padding?

4.	How many multiplications and additions are involved in radix-2 FFT?

5.	Define an IIR filter.

6.	What is aliasing?

7.	List the disadvantages in FIR filter.

8.	What are the characteristics features of rectangular window?

9.	What are the two types of quantization employed in digital system?

10.	How the system output can be brought out of limit cycle?

PART – B (5 x 3 = 15 MARKS)

11.	Determine the fundamental value for the signal x(n)=cos(π/3)n+cos3π4n.

12.	List any four properties of DFT.

13.	Write a short note on prewarping.

14.	Sketch the log magnitude spectrum of hamming window.

15.	What is coefficient quantization error? What is it’s effect?

PART – C (5 x 15 = 75 MARKS)

16.	Determine the convolution of the signals

x(n)	=1		n = -2, 0, 1

2		n= -1,

0		elsewhere	  and   h(n) = δ(n)-δ(n-1)+δ(n-2)-δ(n-3).

(OR)

17.	Find the impulse response for the system described by the difference Equation

y(n) - 1 6y(n-1) -  16 y(n-2) = x(n).

18.	Compute 4 point DFT of a sequence x (n) = {0, 1, 2, 3} using DIT, DIF algorithm.

(OR)

19.	a.	Write the Radix 2 DIF – FFT Algorithm.										(10)

b.	Give the similarities and differences between DIT and DIF algorithm.				(5)

20.	Design a Chebyshev filter using bilinear transformations for the following specifications.

0.8≤|H(ejw )|≤ 1  	 	0 ≤ ω ≤ 0.2π

|H(ejw) |≤ 0.2		0.6 ≤ ω ≤ π

(OR)

[P.T.O]

21.	a.	Realize the system with difference equation y(n) =34y(n-1) - 18 y(n-2) +x(n) + 13 x(n-1) in 	cascade form.																(8)

b.	Realize the system with difference equation y(n) = -0.1 y(n-1) + 0.72 y(n-2) +0.7 x(n) -0.252x(n-2) in parallel form.	 												(7)

22.	Design a FIR low pass filter for the specification with length N= 11.

H d(ejw) = 1 for   -π2  ≤  ω ≤  π2

= 0 for   π2   ≤  ω ≤  π.

Find the values of h(n) and H(z).Plot the magnitude response.

(OR)

23.	A FIR filter approximating the ideal frequency response

H d(ejw) =  e-jαw  for  |ω|  ≤  π6

=   0    for π6  ≤  |ω|  ≤  π.

Determine the filter coefficients for N = 13 using Hamming Window.

24.	Explain the following:

a.	Fixed point representation                                       								(10)

b.	Floating point representation                                                        					(5)

(OR)

25.	Write technical notes on

a.	Principle of scaling.                                    										(8)

b.	Limit Cycle oscillations                            										(7)