# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	FUNDAMENTALS OF ELECTRONIC COMMUNICATION

# Time: 3 hours

# Subject Code:	MT233		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Frequency domain representation of signal is called ___________.

2.	(S/N)dB =

3.	Which oscillator provides a precise and stable frequency?

4.	In super heterodyne mixer, the circuits after the mixer and before detector are fixed tuned to ___________ frequency.

5.	What is modulation index of the AM wave for modulating signal voltage Vm=4V and carrier voltage Vc=5V?

6.	Draw the spectrum of DSB-SC.

7.	What is angle modulation?

8.	Name the circuit used for PAM signal detection.

9.	What is quantization?

10.	Define probability of error.

PART – B (5 x 3 = 15 MARKS)

11.	Two resistors of 50kΩ and 100kΩ are at a room temperature (290K). For a bandwidth of 100kHz, calculate the thermal noise voltage generated by the two resistors in parallel.

12.	What is the role of AGC in super heterodyne receiver?

13.	Calculate the bandwidth of the FM wave for modulation index =4 base band signal frequency fm=15kHz.

14.	State the Sampling Theorem.

15.	Draw the signal waveform of 01001101 for uni-polar and bi-polar synchronous transmission.

PART – C (5 x 15 = 75 MARKS)

16.	Prove mathematically that a mixer converts frequencies. With suitable diagram explain the operation of BJT mixer.

(OR)

17.	What are the different types of power amplifiers? Which amplifier has the maximum efficiency? Which amplifier is preferred for AM transmitters? Explain.

18.	Explain the operation of frequency synthesizers. State its applications.

(OR)

19.	With suitable diagrams explain the importance and operation of double – conversion receivers.

[P.T.O]

20.	For the system shown in the figure

m(t)

y(t)

LPF

0-1KHz

2cos(30000)t

Find the spectrum of y(t)

a.	What type of modulation does this system correspond to?

b.	Draw the spectrum of y(t)

c.	Suggest a method of recovering back the original message m(t) from y(t).

d.	If the LPF is removed what type of modulation does this correspond to? State one 	disadvantage of this scheme over the previous one.

(OR)

21.	Explain the envelope detection method of AM demodulation with necessary diagrams.

22.	Explain with neat diagrams FM detection using balanced slope detector.

(OR)

23.	Explain time division multiplexing with neat diagrams. Why do the transmitter and receiver of Time division multiplexer be synchronized? Give reasons.

24.	What are the different types of pulse modulation techniques? With neat diagram explain the process of pulse code modulation.

(OR)

25.	Discuss how error detection and correction is achieved using Hamming codes.