# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	SIGNAL TRANSMISSION AND RADIATION 			       Time: 3 hours

# Subject Code:	MT238		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	A 300 Ω line is terminated in a load impedance of 100+j200Ω. Determine the voltage reflection co -efficient.

2.	Draw the reactance curve for a short circuited line.

3.	Write the Wave impedance equation for TE wave.

4.	Define dominant mode in Rectangular waveguide.

5.	What is meant by superfraction?

6.	Define critical frequency.

7.	Define Polarization of a wave.

8.	Mention the different types of Non-resonant antennas.

9.	Define Numerical aperture.

10.	What are called leaky modes?

PART – B (5 x 3 = 15 MARKS)

11.	Calculate the voltage standing-wave ratio for a lossless line of characteristic impedance 50Ω when it is terminated in a 100Ω resistive and 30-j50Ω.

12.	A rectangular waveguide has a broad wall dimension of.0.9 inch and is fed by a 10GHz carrier from a co-axial cable. Determine whether a TE10 wave will be propagated and if so, find its guide wavelength phase and group velocities.

13.	Write short notes on Extremely Low Frequency propagation.

14.	In a VHF mobile radio system, the base station transmits 100W at 150MHz and the antenna is 20m above ground.  The transmitting antenna is half-wave dipole for which the gain is 1.64. Calculate the field strength at a receiving antenna of height 2m at a distance of 40km.

15.	An optic fiber has a core diameter of 50μm and is used at a light wavelength of 0.8μm. Find its ν number and the approximate number of modes will support.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain how standing waves may be set up on a transmission line, and state the 	relationship between voltage and current standing waves for a line whose characteristics 	impedance is purely resistive.                                                                          			(8)

b.	Measurements on a 50Ω slotted line gave the following results: VSWR=3.0, distance from load to first voltage minimum=0.4λ.  Determine, using a smith chart, the value of the load impedance. 																	(7)

(OR)

17.	a.	Sketch the form of trace expected on a TDR measuring set when applied to a line 	terminated in a series RL circuit. Give equations for the asymptotic values on the trace.(8)

b.	The primary constants for a transmission line are   R=0.5Ω/m, L=250nH/m, C=100pF/m and g=10-6S/ m. Calculate the characteristic Impedance, attenuation co-efficient and the phase shift c0-efficient at a frequency of 20MHz. 								(7)

[P.T.O]

18.	Elaborate the structure and importance of Micro strip lines.

(OR)

19.	Sketch and explain in detail about rectangular waveguides.

20.	Explain what is meant by the gyro frequency.  Why are frequencies in the region of the gyro 	frequency not suitable for ionospheric transmissions?

(OR)

21.	Explain the difference between the surface wave and ground wave for radio transmissions in 	the frequency range from 300 KHz to 2 MHz.

22.	Discuss in detail about Helical and Log periodic antenna.

(OR)

23.	Explain about Microwave antennas.

24.	Explain in detail how Avalanche diode works. Also compare the sensitivity of Avalanche and Pin diode.

(OR)

25.	What precautions should be taken when installing or repairing fiber equipment, connectors or splices? Explain why these precautions are so important in terms of impairment of transmission.