# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MICROPROCESSORS AND INTERFACING 			       Time: 3 hours

# Subject Code:	09IT206		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Name the two pointer registers.

2.	What name is given to the part of the I/O address space from 000016 through 00FF16?

3.	Why does the instruction MOV CL, AX result in an error when it is assembled?

4.	What are the two different types of jump operation?

5.	Define time state of a processor.

6.	What are the word lengths of the 8088’s address bus and data bus?

7.	The source or destination register with in the 8255 is selected by a ________ bit_______ select 	code.

8.	What is baud rate?

9.	What does STI stands for?

10.	The 8086 and 8088 microcomputer systems are capable of implementing up to ________ software interrupts.

PART – B (5 x 3 = 15 MARKS)

11.	Write a note on stack.

12.	List the addition subgroup instructions.

13.	List the Input / Output instructions.

14.	Draw the block diagram of 8237A DMA controller.

15.	Write a note on NMI.

PART – C (5 x 15 = 75 MARKS)

16.	Draw the block diagram of 8086 and explain its architecture.

(OR)

17.	a.	Draw the software model of 8086 and explain.                         						(8)

b.	How is a physical address generated? Explain with examples.    					(7)

18.	List the 6 groups of instruction in 8088/8086 instruction set. Explain them with examples.

(OR)

19.	Make a list of the addressing modes available on the 8086 and explain them with examples.

20.	Draw the diagram for 8086 maximum mode system configuration and explain.

(OR)

21.	Draw the memory interface block diagram and explain its operation.

22.	Explain 8254 and its interface with a block diagram.

(OR)

23.	Explain programmable communication interface controller with neat diagram.

24.	Explain about external hardware – interrupt interface.

(OR)

25.	Draw the internal architecture of 8259 and explain.