# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	OPERATIONS RESEARCH TECHNIQUES 				 Time: 3 hours

# Subject Code:	09MA304		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Consider the data of a project as shown below

| Activity | Normal time (weeks) | Normal cost (`.) | Crash time (weeks) | Crash cost (`.) |
| --- | --- | --- | --- | --- |
| 1-2 | 13 | 700 | 9 | 900 |
| 1-3 | 5 | 400 | 4 | 460 |
| 1-4 | 7 | 600 | 4 | 810 |
| 2-5 | 12 | 800 | 11 | 865 |
| 3-2 | 6 | 900 | 4 | 1130 |
| 3-4 | 5 | 1000 | 3 | 1180 |
| 4-5 | 9 | 1500 | 6 | 1800 |

If the indirect cost per week is Rs. 160, find the optimal crashed project completion time. (Up to third Iterations) 													(14)

b.	What are the types of Simulation? Explain them in brief. 						(6)

2.	a.	Solve the following LPP using graphical method: 							(8)

Minimize Z = 2x1 + 3x2

Subject to 	2x1 + 3x2 ≥ 6

7x1 + x2 ≥ 14

x1 and x2 ≥ 0.

b.	Solve the following LPP using simplex method: 								(12)

Maximize Z = 10x1 + 15x2 + 20x3

Subject to 	2x1 + 4x2 + 6x3 ≤ 24

3x1 + 9x2 + 6x3 ≤ 30

x1, x2and x3 ≥ 0.

(OR)

3.	a.	Define the following: (i) Feasible (ii) Unbounded and (iii) optimal solution. 		(6)

b.	Solve the following LPP using Big M method: 								(14)

Maximize Z = 10x1 + 15x2 + 20x3

Subject to 	2x1 + 4x2 + 6x3 ≥ 24

3x1 + 9x2 + 6x3 ≥ 30

x1, x2and x3 ≥ 0.

4.	Solve the following LPP using dual simplex method:

Minimize Z = 40x1 + 30x2 + 25x3

Subject to 	4x1 + 2x2+ 5x3 ≥ 30

3x1 + 6x2 + x3 ≥ 20

x1 + 3x2 + 6x3 ≥ 36

x1, x2and x3 ≥ 0.

(OR)

[P.T.O]

5.	Solve the following transportation problem for optimal distribution using Vogel’s approximation method.

| Plant | I | II | III | IV | V | Supply |
| --- | --- | --- | --- | --- | --- | --- |
| I | 10 | 2 | 16 | 14 | 10 | 300 |
| II | 6 | 18 | 12 | 13 | 16 | 500 |
| III | 8 | 4 | 14 | 12 | 10 | 825 |
| IV | 14 | 22 | 20 | 8 | 18 | 375 |
| Demand | 350 | 400 | 250 | 150 | 400 |  |

6.	The flight timings between two cities X and Y are as given below respectively. The minimum layover time of any crew in either of the cities is 2 hours. Determine the base city for each crew so that the sum of the layover times of all crew members in non-base cities is minimized.

| Flight Timings from City X to Y |  |  | Flight Timings from City Y to X |  |  |
| --- | --- | --- | --- | --- | --- |
| Flight Number | Departure Time (from City X) | Arrival time ( to city Y) | Flight Number | Departure Time (from City Y) | Arrival time ( to city X) |
| 101 | 5 a.m | 6.15 a.m | 201 | 6.30 a.m | 7.30 a.m |
| 102 | 9 a.m | 10.15 a.m | 202 | 9.00 a.m | 10.00 a.m |
| 103 | 1 p.m | 2.15 p.m | 203 | 3.30 p.m | 4.30 p.m |
| 104 | 6 p.m | 7.15 p.m | 204 | 10.00 p.m | 11.00 p.m |

(OR)

7.	a.	Explain steps involved in Johnson’s Algorithm for n jobs and two machines problem. (6)

b.	Consider a two jobs and m machines job shop scheduling problem given below. Use graphical method to minimize the time needed to process the following jobs on the machines shown. Also calculate the total time elapsed to complete both jobs. 		(14)

| Job 1 |  | Job 2 |  |
| --- | --- | --- | --- |
| Sequence | Time (hrs) | Sequence | Time (hrs) |
| A | 5 | B | 7 |
| B | 6 | C | 6 |
| C | 4 | A | 5 |
| D | 8 | D | 4 |
| E | 4 | E | 8 |

8.	a.	If people arrive to purchase cinema tickets at the average rate of 6 per minute, it takes an 	average of 7.5 seconds to purchase a ticket. If a person arrives 2 min before the picture 	starts and if it takes exactly 1.5 min to reach the correct seat after purchasing the ticket,  	(i) Can he expect to be seated for the start of the picture? (ii) What is the probability that 	he will be seated for the start of the picture? 										(10)

b.	There are three typists in an office. Each typist can type an average of 6 letters per hour. If letters arrive for being typed at the rate of 15 letters per hour, (i) what fraction of the time all the typists will be busy? (ii) What is the average number of letters waiting to be typed? 																		(10)

(OR)

9.	a.	Given an average arrival rate of 20 per hour, is it better for a customer to get service at a 	single channel with mean service rate of 22 customers per hour or at one of two channels 	in parallel with mean service rate of 11 customers per hour for each channels, Assume 	both the queues to be of poisson type. 											(10)

b.	A 2-person barber shop has 5 chairs to accommodate waiting customers. Potential customers, who arrive when all 5 chairs are full, leave without entering barber shop. Customers arrive at the average rate of 4 per hour and spend an average of 12 min in the barber’s chair. Compute P0 and E (Nq). 											(10)