# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ENGINEERING THERMODYNAMICS 					       Time: 3 hours

# Subject Code:	09ME202	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

#### Use of Steam Tables permitted

PART – A (10 x 1 = 10 MARKS)

1.	What do you understand by macroscopic view point?

2.	What is zeroth law of thermodynamics?

3.	Entropy is a function of _________which shows the possibility of conversion of heat into work.

4.	Write the expression for thermal efficiency of a cyclic heat engine.

5.	A pure liquid at a given pressure will transform into vapour only at a particular temperature, known as _________ which is a function of pressure.

6.	What is ‘Degree’ of superheat?

7.	The ratio pV / RT is called _________ factor.

8.	State the Avogadro’s law.

9.	Rankine cycle comprises two isentropic processes and two _________ processes.

10.	The efficiency of engine using air as the working medium is known as _________.

PART – B (5 x 3 = 15 MARKS)

11.	What are intensive and extensive properties? Give suitable examples.

12.	What is a reversed heat engine? Give examples.

13.	What is critical state? Write the values of critical pressure, critical temperature and critical volume of water.

14.	What is the ideal equation of state? Determine the specific volume of methane gas at10 MPa, 250 K and the value of R is 0.5182 kJ/kg K.

15.	Write any three assumptions made in the analysis of air standard cycles.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain the various types of thermodynamic systems.                                         		(5)

b.	A fluid is confined in a cylinder by a spring-loaded, frictionless piston so that the pressure in the fluid is a linear function of the volume (p = a + bV). The internal energy of the fluid is given by the following equation U = 34 + 3.15 pV, where U is in kJ, p in kPa, and V in m3. If the fluid changes from an initial state of 170 kPa, 0.03 m3 to a final state of 400 kPa, 0.06m3, with no work other than that done on the piston, find the direction and magnitude of the work and heat transfer.													(10)

(OR)

17.	a.	State and explain the first law for a closed system undergoing a change of state.		(5)

b.	In a gas turbine the gas enters at the rate of 5 kg/s with a velocity of 50 m/s and enthalpy of 900 kJ/kg and leaves the turbine with a velocity of 150 m/s and enthalpy of 400 kJ/kg. The loss of heat from the gases to the surroundings is 25 kJ/kg. Assume for gas R = 0.285 kJ/kg K and Cp = 1.004 kJ/kg K and the inlet conditions to be at 100 kPa and 270C. Determine the power output of the turbine and the diameter of the inlet pipe.					(10)

[P.T.O]

18.	a.	Give the Kelvin – Planck and Clausius statements based on the second law.        		(5)

b.	Explain the Carnot heat engine cycle executed by a stationary system.            		(10)

(OR)

19.	a.	State and explain the Carnot’s theorem with suitable block diagram.                 		(8)

b.	A refrigeration plant for a food store maintains a temperature of – 10C. The ambient air temperature is 32C. If heat leaks into the plant at the continuous rate of 2 kJ/s, what is the least power necessary to pump this heat out continuously. 							(7)

20.	a.	Explain the term ‘quality of steam’. Differentiate between wet, dry-saturated and 	superheated steam.                                                                                               		(5)

b.	Find the dryness fraction, specific volume and internal energy of steam at 7 bar and enthalpy 2550 kJ/kg.                                                                                    			(10)

(OR)

21.	a.	Draw the phase equilibrium for a pure substance on T – s diagram and explain briefly. (7)

b.	Steam initially at 1.5 MPa, 3000C expands reversibly and adiabatically in a steam turbine to 400C. Determine the ideal work output of the turbine per kg of steam.			(8)

22.	a.	Write down the Van der walls equation of state. How does it differ from the ideal gas 	equation of state?                                                                                            			(5)

b.	Write short notes on compressibility factor and compressibility chart.               		(5)

c.	Define ‘calorific value of fuel’. Differentiate between H.C.V. and L.C.V. of a fuel.    	(5)

(OR)

23.	Describe with a neat diagram the bomb calorimeter used for finding calorific value of solid and liquid fuels.

24.	Draw the p – v and T – s diagrams of air standard diesel cycle and derive the expression for air standard efficiency.

(OR)

25.	An engine working on the otto cycle is supplied with air at 0.1 MPa, 35C. The compression ratio is 8. Heat supplied is 1200 kJ/kg. Calculate the maximum pressure and temperature of the cycle, the cycle efficiency and the mean effective pressure. For air take Cp=1.005 kJ/kg K, Cv=0.718 kJ/kg K and R = 0.287 kJ/kg K.