# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	THERMAL ENGINEERING 							       Time: 3 hours

# Subject Code:	09ME203	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

(Use of Standard Thermodynamic Tables, Mollier Diagram, Psychometric Chart and Refrigerant Property Tables are permitted)

PART – A (10 x 1 = 10 MARKS)

1.	The thermal efficiency of a two stroke petrol engine is _______ than a four stroke petrol engine.

2.	What is meant by effective pressure?

3.	What is the function of steam nozzle?

4.	What are the different methods of compounding?

5.	Write any one application of rotary compressor.

6.	Ideal intermediate pressure P2 for two stage reciprocating air compressor is given by ______.

7.	Define COP of refrigerator.

8.	One ton of refrigeration is equivalent to _______kW.

9.	Define sensible heat factor.

10.	Define specific humidity.

PART – B (5 x 3 = 15 MARKS)

11.	What are the effects of knocking?

12.	What are the effects of friction on flow through steam nozzle?

13.	Draw p-v and T-S diagram for a single stage reciprocating compressor without clearance volume.

14.	What is sub cooling in refrigeration system?

15.	Write short notes on window air conditioning system.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain the water cooling system of an IC engine with neat sketch.				       	(7)

b.	Draw and explain the port timing diagram of a two-stroke diesel engine.			(8)

(OR)

17.	An engine develops 20 kW of brake power with a mechanical efficiency of 80% consuming petrol at the rate of 5.8 kg/hr.  Calculate

a.	Brake thermal efficiency 		b.	Specific fuel consumption and

c.	Indicated thermal efficiency if the calorific value of the fuel is 42,000 kJ/kg.

18.	a.	Compare impulse and reaction turbine. 											(5)

b.	Explain reheating Rankine cycle with neat sketch	.							     	(10)

(OR)

19.	Dry saturated steam at 10 bar is expanded isentropically in a nozzle to 0.1 bar. Using steam tables, find the dryness fraction of the steam at exit. Also find the velocity of steam leaving the nozzle when (i) initial velocity is negligible and 	(ii) initial velocity of the steam is 135 m/s.

[P.T.O]

20.	A two stage reciprocating air compressor takes in 2.82 m3 of air per minute at a pressure of 1.05 bar and temperature 295 K. It delivers the air at 8.44 bar. The 	compression is carried out in each cylinder according to law pv1.2=C.  The air is cooled to its initial temperature in intercooler. Neglecting clearance, find the minimum power required to drive the compressor.

(OR)

21.	A single stage, single acting reciprocating air compressor has 0.3 m bore and 0.4 m stroke runs at 400 rpm.  The suction pressure is 1 bar at temperature 300 K and delivery pressure is 5 bar. Find the power required to run it, if compression is isothermal, compression follows the law pv1.3=C and compression is isentropic.

22.	With a neat flow diagram, explain the working of a vapour compression refrigeration system.

(OR)

23.	A Vapour compression refrigerator uses the refrigerant R12 and operates between the pressures 2.5 bar and 9 bar. The compression is isentropic and there is no under cooling in the compressor. The vapour is dry saturated condition at the beginning of the compression. Calculate the theoretical COP of the refrigerator. Take Cp for superheated vapour at 9 bar is 0.67 kJ/kg K. The properties of refrigerant are as follows.

| Pressure bar | Temperature º C | Enthalpy (kJ/kg) |  | Entropy of saturated vapour (kJ/kg K) |
| --- | --- | --- | --- | --- |
|  |  | Liquid | Vapour |  |
| 9 | 36 | 456.4 | 585.3 | 4.74 |
| 2.5 | -7 | 412.4 | 570.3 | 4.76 |

24.	a.	Explain about the adiabatic mixing of two air streams. 							(8)

b.	Explain about the evaporative cooling with sketch. 								(7)

(OR)

25.	An office with a seating capacity of 50 persons is to be air-conditioned with the following conditions.

Outdoor conditions		:	10C DBT and 8C WBT

Desired indoor conditions	:	20C DBT and 60% RH

Amount of air circulation	:	0.3 m3 / min / person

The desired condition is achieved first by heating and then by adiabatic humidifying. Determine

a.	Heating capacity of the coil in kW and the surface temperature required if the bypass factor of the coil is 0.32.

b.	The capacity of the humidifier.