# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	ADVANCED THERMODYNAMICS  					 Time: 3 hours

# Subject Code:	09ME302		         		        				 Maximum Marks: 100

(Use of steam table and compressibility chart permitted)

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Discuss the assumptions made in kinetic theory of gases.	                               	(10)

b.	Calculate the r.m.s. speed for oxygen at 300 K.  What is the mean translational kinetic 	energy of a molecule of oxygen? 	                                                       			(10)

2.	a.	Calculate the decrease in available energy when 25 kg of water at 950C mix with 35 kg 	of water at 350C, the pressure being taken as constant and the temperature of the 	surroundings being 150C (cp  of water = 4.2 kJ/kg K)  	                               		(12)

b.	Define the second law efficiency. How is it different from the first law efficiency in the case of a simple power plant?   	                                                         			(8)

(OR)

3.	a.	Explain the significance of Helmholtz function in thermodynamics.                    	(8)

b.	One kg of water at 273 K is brought into contact with a heat reservoir at 373 K. When the water has reached 373 K, find the entropy change of the water, of the heat reservoir and of the universe.  If water is heated from 273 K to 373 K by first bringing it in contact with a reservoir at 323 K and then with a reservoir at 373 K, what will be the entropy change of the universe?  	  	                               							(12)

4.	Write down the first and second Tds equations, and derive the expression for the difference in heat capacities cp and cv

(OR)

5.	a.	Derive the Clausius – Clapeyron equation,  					(10)

b.	Obtain the condition of thermal stability for a system.	                               		(10)

6.	a.	Estimate the specific volume and density of nitrogen at 170 K and 4 MPa using 	compressibility charts. Also calculate the pressure at v=0.004 m3/kg and T=170 K for 	nitrogen.  Take c=3.398 MPa, Tc=126.2 K and vc=0.0899 m3/kg mol.         		(10)

b.	Explain the significance of the generalized compressibility factor.	                  	(10)

(OR)

7.	a.	Explain the following terms: (i) Fugacity and (ii) Activity 	                               	(10)

b.	Explain the following terms associated with combustion process:

i) Reactants and products 		(ii) Stoichiometric air  	(iii) Excess air.		(10)

8.	An analysis is made of the products of combustion when methane (CH4) is burned with atmospheric air.  The mole fractions of the dry products (those other than water) are as follows:  CO2=9% O2 = 1.50%, CO=1.20% and N2=88.3%.  Calculate the percentage of theoretical air used in the combustion process.

(OR)

9.	a.	Explain the collision frequency and mean free path of gas particles. 	      		(10)

b.	Compute the most probable speed, the mean speed, and the root-mean square speed for 	Helium at 00C.                                                                                      			(10)