# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	PROPULSION ENGINEERING 							 Time: 3 hours

# Subject Code:	09ME316		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	With the aid of the schematic diagram and thermodynamic cycle, explain the working of 	a turboprop engine.						    								(15)

b.	What do you understand by the term air-breathing engines? 		     			(5)

2.	a.	Determine the number of radial impeller vanes using Stanitz formulae for a centrifugal 	which requires 180 kJ of power input per unit mass flow rate and is running at 15000 	rpm. Guide vanes at inlet give the air a prewhirl of 25 to the axial direction at all radii 	and the mean dia of eye is 250 mm. Impeller tip dia is 600 mm. The absolute air velocity 	at inlet is 150 m/s. 			    											(15)

b.	Why are diffusers necessary in a centrifugal compressor? 			    			(5)

(OR)

3.	a.	Explain the flow through a compressor and bring out the details of various losses in an 	axial flow compressor. 							   						(15)

b.	Define degree of reaction and derive an expression for an axial flow compressor.	(5)

4.	a.	What are the various factors those affect the combustion chamber performance? 	Explain.

(15)

b.	What are the various possibilities of combustion chamber arrangements?	 	(5)

(OR)

5.	a.	In a single – stage impulse turbine the nozzle discharges the fluid on to the blades at an 	angle of 65 to the axial direction and the fluid leaves the blades with an absolute 	velocity of 300 m/s at an angle of 30  to the axial direction. If the blades have equal inlet 	ant outlet angles and there is no axial thrust, estimate the blade angle, power produced 	per kg/s of the fluid and the blade efficiency.	    								(15)

b.	Explain the principle of operation of a turbine.  What are the primary parts of turbine?

(5)

6.	a.	How does water-alcohol mixture injection help thrust augmentation in afterburner	arrangement? Explain. 							   						(15)

b.	What are the main components in the afterburner arrangements?                     	(5)

(OR)

7.	a.	Briefly explain the supersonic inlets. 					  	    				(8)

b.	Summarize the functions of an exhaust nozzle. 			    	    				(7)

c.	What is the purpose of a diffuser in the inlet? 				    				(5)

8.	a.	What is an equilibrium diagram? Explain with sketches the method of finding the 	equilibrium points. 							 							(15)

b.	Mention the various assumptions in the general matching trends. 		  		(5)

(OR)

9.	a.	Explain the general matching procedure for a two-spool turbojet engine.	 	(15)

b.	Define match point and explain the effect of exit nozzle area on the match point.	(5)