# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	FINITE ELEMENT ANALYSIS 							 Time: 3 hours

# Subject Code:	10ME302		         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Solve the following system of equation by using Gauss Elimination method:

x – y + z  = 1

-3x + 2y - 3z  =  -6

2x - 5y + 4z  = 5

(OR)

2.	Using Power method, find the largest Eigen value and corresponding Eigen vector of matrix

A =

3.	The following differential equation is available for a physical phenomenon d2y /dx2 + 50 = 0, 0 ≤ x ≤ 10, Trial function is y = a1 (10x – x2).  Boundary conditions are y(0) = 0 and y(10)=0.  Find the value of parameter  a1 by the following method:

a.	Point collocation 		b.	Least square method		c.	Sub domain method

d. 	Galerkin’s method

(OR)

4.	Derive the shape functions for a 2D bilinear rectangular element using local co-ordinate system.

5.	Compulsory;

Using two finite elements, find the stress distribution in a uniformly tapering bar of circular cross-sectional area 3cm2 and 2cm2 at their ends, length 100mm, subjected to an axial tensile load of 50N at smaller end and fixed at larger end. Take the value of young’s modulus 2x105 N/mm2.

6.	Using Galerkin’s method, evaluate the Stiffness matrix for a 2D triangular element suitable for any field problems.

(OR)

7.	Solve the torsion problem for a square shaft of 1cm2 and calculate the shear stress for any one element. Use 2gθ=2790.

8.	Calculate the temperature distribution in a 1D fin with the physical properties as shown in fig.1. The fin is rectangular in shape, 8cm long, 4cm wide and 1cm thick. Assume that convection heat loss occurs from the right end of the fin.  (use 4 element idealization)

80C

1 cm

L = 8 cm

4 cm

(OR)

9.	Briefly explain Boundary element method and discuss its advantages, disadvantages over Finite element method.