# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	GAS DYNAMICS AND JET PROPULSION 				       Time: 3 hours

# Subject Code:	ME217		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Stagnation state.

2.	What do you mean by supersonic flow region?

3.	State the purpose of a nozzle.

4.	The flow before and after the shock may still be ______________.

5.	What is Rayleigh flow?

6.	Define Isothermal process.

7.	Write any one application of shock wave.

8.	Fannoline describes an __________ process in a constant area duct with friction.

9.	Define thrust.

10.	What are Solid Propellant Rockets?

PART – B (5 x 3 = 15 MARKS)

11.	Write short notes about Crocco number.

12.	What is meant by strength of a shock wave?

13.	Write any three assumptions used in Fanno flow.

14.	Write about Fanno line.

15.	What is reheating in Turbine engines?

PART – C (5 x 15 = 75 MARKS)

16.	Air (Cp = 1.05kJ/kgK, γ = 1.38) at p1 = 3 bar and T1 = 500 K flows with a velocity of 200 m/s in a 30 cm diameter duct. Calculate

a.	Mass flow rate		b.	Stagnation temperature	c.	Mach number

d.	Stagnation pressure values assuming the flow as compressible and Incompressible 	respectively.															      (5+3+3+4)

(OR)

17.	An air jet (γ = 1.4, R = 287 J/kg K) at 400 K has sonic velocity. Determine			      (5 x 3)

a.	The velocity of sound at 400K		b.	Velocity of sound at stagnation conditions

c.	Maximum velocity of the jet		d.	Stagnation enthalpy	 e.	Croccco number

18.	A conical diffuser has entry and exit diameters of 15 cm and 30 cm respectively. The Pressure, temperature and velocity of air at entry are 0.69 bar, 340 K and 180 m/s respectively. Determine

a.	The exit pressure		b.	The exit velocity									      (3x 5)

c.	The force exerted on the diffuser walls. Assume isentropic flow, γ = 1.4, Cp = 1 KJ/kg K

(OR)

[P.T.O]

19.	Air flows in a duct has a velocity of 300 m/s, pressure 1 bar and temperature 290 K.

Taking γ = 1.4, R = 287 J/kg K. Determine

a.	stagnation pressure and temperature   											(5)

b.	velocity of sound in the stagnation and dynamic conditions						(5)

c.	stagnation pressure assuming constant density									(5)

20.	Air at P0 = 10 bar, T0 = 400 K is supplied to a 50 mm diameter pipe. The friction factor for the pipe surface is 0.002. If the Mach number changes from 3.0 at the entry to 1.0 at the exit. Determine: 	a.	the length of the pipe			b.	mass flow rate   				       (7+8)

(OR)

21.	The Mach number at the exit of the combustion chamber is 0.9. The ratio of stagnation temperatures at exit and entry are 3.74. If the pressure and temperature of the gas at exit are 2.5 bar and 723K respectively. γ = 1.4, Cp =1.218 KJ/kg K  Determine

a.	Mach number, pressure and temperature of the gas at entry					   (4+4+4)

b.	The heat supplied per kg of the gas												(3)

22.	Derive the Prandtl Meyer relation.

(OR)

23.	a.	Define normal and oblique shocks												(2+2)

b.	A gas (γ = 1.4, R = 287 J/kg K) at a Mach number of 1.8, P = 0.8 bar and T = 373 K passes through a normal shock. Determine its density after the shock. Compare this with an isentropic compression through the same pressure ratio.							(6+5)

24.	Explain briefly about the liquid propellant rockets.

(OR)

25.	An aircraft flies at 960 km/hr. One of its turbojet engines takes in 40 kg/s of air and expands the gases to the ambient pressure. The air-fuel ratio is 50 and the Lower calorific value of the fuel is 43 MJ / kg. For maximum thrust power Determine

a.	Jet velocity		b.	Thrust		c.	specific thrust		d.	thrust power

e.	propulsive, thermal and overall efficiencies.									        (5x3)