# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	HEAT ENGINES AND FLUID MACHINERY 			       Time: 3 hours

# Subject Code:	ME223		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The specific volume is the reciprocal of ___________.

2.	The specific gravity of mercury is ___________.

3.	___________ is the ratio of power available at the shaft of the turbine to the power supplied by the water at the inlet of the turbine.

4.	If at the inlet of the turbine, the energy available is only kinetic energy, the turbine is known as ___________ turbine.

5.	___________ is the energy possessed by a body because of its height above the earth surface.

6.	___________ law states that energy can neither be created nor be destroyed.

7.	For the same temperature source and sink the thermal efficiency of Carnot cycle is equal to that of ___________ cycle.

8.	For a fixed compression ratio, the thermal efficiency of ___________ when operating on standard air is more than that when operating on CO2.

9.	Thermal ___________ is a mechanism of heat propogation from a region of higher temperature to a region of low temperature within a medium.

10.	A body that reflects all the incident thermal radiation is called a ___________body or a absolutely white body.

PART – B (5 x 3 = 15 MARKS)

11.	The right limb of a simple U – tube manometer containing mercury is open to the atmosphere while the left limb is connected to a pipe in which a fluid of specific gravity 0.9 is flowing. The centre of the pipe is 12 cms below the level of the mercury of the right limb. Find the pressure of the fluid in the pipe, if the difference of mercury level in the two limbs is 20 cms.

12.	What is a draft tube? Why it is used in a reaction turbine?

13.	Define Kelvin Planck statement.

14.	What is mean effective pressure?

15.	What is meant by absorptivity, reflectivity and transmissivity?

PART – C (5 x 15 = 75 MARKS)

16.	The cylinder of a single-acting reciprocating pump is 15 cm in diameter and   30 cm in stroke.  The pump is running at 30 r.p.m. and discharge water to a   height of 12 m. The diameter and length of the delivery pipe are 10 cm and 30 m respectively. If a large air vessel is fitted in the   delivery pipe at a distance of 2m from the centre of the pump, find the pressure head in the cylinder. (i) At the beginning of the stroke, and (ii) In the middle of the delivery stroke. Take

f = .01.

(OR)

17.	Explain the construction and working principle of gear oil pump in detail with neat sketch.

[P.T.O]

18.	A Francis turbine with an overall efficiency of 75% is required to produce 148.25 kW power. It is working under a head of 7.62 m. The peripheral velocity = 0.26and the radical velocity of flow at inlet is 0.96√2gH. The wheel runs at 150 r.p.m and the hydraulic losses in the turbine are 22% of the available energy. Assuming the radial discharge, determine: (i) The guide blade angle, (ii) The wheel vane angle at inlet, (iii) Diameter of the wheel at inlet, and (iv) Width of wheel at inlet.

(OR)

19.	A Kaplan turbine working under a head of 20m develops 11772 kW shaft power. The outer diameter of the runner is 3.5 m and hub diameter 1.75m. The guide blade angle at the extreme edge of the runner is 35°. The hydraulic and overall efficiencies of the turbines are 88% and 84% respectively. If the velocity of the whirl is zero at outlet, determine: (i) Runner vane angles at inlet and outlet at the extreme edge of the runner, and (ii) Speed of the turbine.

20.	Air flows steadily at the rate of 22.68 kg/min through an air compressor entering at 6.1m/s velocity,1.02 bar pressure and 0.845m3/kg and leaving at 4.6m/s, 7.03 bar pressure and 0.163m3/kg. The internal energy of fluid leaving is 88.2 kJ/kg greater than that of the air entering. Cooling water in the compressor jacket absorbs heat from the air at the rate of 3693 kJ/min.

(OR)

21.	State and prove Carnot’s Theorem.

22.	The following observations were made during the trial of a Diesel engine. The power developed by the engine is used for driving a D.C. generator. The output of the generator was 200 amperes at 200 volts. The efficiency of the generator being 80%. The quantity of the fuel supplied to the engine was 11 kg/h. Calorific value of fuel being 42500kJ/kg. The air fuel ratio was 19:1. The exhaust gases were passed through a exhaust gas calorimeter for which the observations were as under.  Water circulated through exhaustgas calorimeter = 575 litres /h. Temperature rise of water through calorimeter = 35°C. Temperature of exhaust gases at exit from calorimeter = 97°C. Ambient temperature = 17°C. If the specific heat of exhaust gases is 1.05kJ/kg K draw up the heat balance sheet on minute basis. Heat lost to jacket cooling water is 30 % of total heat supplied.

(OR)

23.	In an air standard diesel cycle, the compression ratio is 16, cut off takes place at 5% of the stroke, the compressioncommences from 1 bar and 28°C. The air flow rate is kg/min. Calculate (i) Indicated power (ii) Air standard efficiency (iii) Mean effective pressure. Assume Cp = 1.005 KJ/g°K. Cv = 0.718 J/kg°K.

24.	a.	State the Fourier law of heat conduction. 										(3)

b.	Derive an expression for steady heat conduction through a plane wall of thickness L

maintained at its two surfaces at temperature T1 and T2 respectively.				(12)

(OR)

25.	A Steel tube of 80mm internal diameter and 6mm wall thickness has 8 longitudinal fins of 1.5mm thickness. Each fin extends 35 mm from the pipe wall and thermal conductivity of fin material is 50 W/m-deg. The wall temperature, the ambient temperature and surface heat transfer coefficient are stated to be 150°C, 25°C and 80 W/m2 – deg. Make calculations for the percentage increase in the rate of heat transfers  for the finned tube over the plain tube.