# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DESIGN OF MECHANICAL TRANSMISSION SYSTEM	       Time: 3 hours

# Subject Code:	ME231		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Which loads are supported by thrust bearings?

2.	List any two belt material.

3.	Give any one application of steel wire nope.

4.	Define module.

5.	What is the function of skew gear?

6.	Where are worm gear boxes used?

7.	In gear box, for a set of gears if the centre distance and module are same, then the sum of teeth of engaging pair will be __________.

8.	What are the types of speed reducers?

9.	Give any one function of ratchet mechanism.

10.	What is a power screw?

PART – B (5 x 3 = 15 MARKS)

11.	What are the differences in the characteristics of sliding contact and rolling contact bearings?

12.	What are the common failures in Gear tooth?

13.	Give the classification of Bevel gears.

14.	What are the components of a gear box?

15.	Define indexing. State the working of Geneva mechanism.

PART – C (5 x 15 = 75 MARKS)

16.	Design a journal bearing for a centrifugal pump from the following data:

Load  on  the journal  =  20,000 N ; Speed of  the journal = 900 r.p.m.; Type of oil is SAE 10, for  which  the absolute viscosity at 55°C is 0.017 kg/m-s; Ambient  temperature of oil is 15.5°C ; Maximum bearing pressure for the pump is 1.5 N/mm2.  Calculate also mass of the lubricating oil required for the artificial cooling, if rise of temperature of oil be limited to 10ºC. Heat dissipation coefficient = 1232W/m2/°C.[Take l/d =1.6]

(OR)

17.	Design a V belt drive to the following specifications:

Power to be transmitted             :     75 K.W

Speed of driving wheel              :     1400 rpm

Speed of driven wheel               :      400 rpm

Diameter of driving wheel         :     300 mm

Centre distance                          :      2500 mm

Service                                       :     16 hours/day

[P.T.O]

18.	Design a chain drive to actuate a compressor from a 10 kW electric motor at 960 rpm.  The compressor speed is to be 350rpm.  Minimum center distance should be 0.5.m Motor is mounted on an auxiliary bed. Compressor is to work for 8 hours/day.

(OR)

19.	Design a pair of helical gears to transmit 10kW at 1000rpm of the pinion.  Reduction ratio of 5 is required. Give details of the drive in a tabular form.

20.	Design a bevel gears drive to transmit 7 kW at 1600rpm for the following data.

Gear ratio =3

Material for pinion and gear = C 45 steel

Life =10,000 hours.

(OR)

21.	Design the worm gear drive to transmit 20 kW at 1440rpm.  Speed ratio is 24. The worm is to be of hardened steel and the wheel is made of chilled phosphor bronze. Considering wear and strength, design worm and worm wheel.

22.	Design the layout of a 12 speed gear box for a lathe. The minimum and maximum speeds are 100 and 1200 rpm.  Power is 5 kW from 1440 rpm induction motor. Construct the speed diagram using a standard speed ratio.

(OR)

23.	Design a set of a speed cones to obtain the following speeds by a crossed belt drive for a machine tool from a counter shaft running at 110 rpm.

50 rpm, 70 rpm, 90 rpm

Centre distance between the axes of counter shaft and the spindle of the machine tool is 3 metres.  Smallest pulley diameter may be 160 mm.

24.	Design a ratchet and pawl arrangement for a braking torque of 200Nm, using safe bending stress of 40Mpa for the wheel and pawl pin. Choose the number of teeth on ratchet wheel as 16.

(OR)

25.	A  Radical cam rotating at 150 rpm is driving a 15mm diameter translating roller follower to produce the following motions, rise of 25 mm with SHM in 150° of cam rotation, dwell for 60°, fall of 25 mm with SHM in120° of cam rotation, and dwell for remaining 30°.   Construct the cam contour with the pressure angle limited to approximately 20°