# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICS OF MACHINES - II 						       Time: 3 hours

# Subject Code:	ME255		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	State D’Alembert’s principle.

2.	What is meant by free body diagram?

3.	What is the function of flywheel?

4.	Define maximum fluctuation of energy.

5.	What is balancing of rotating masses?

6.	What is hammer blow?

7.	Define magnification factor.

8.	What is meant by logarithmic decrement?

9.	Define gear ratio.

10.	Write expression for equivalent length of shaft.

PART – B (5 x 3 = 15 MARKS)

11.	What are the conditions for dynamic balance?

12.	Define coefficient of fluctuation of energy and coefficient of fluctuation of speed.

13.	What is meant by swaying couple?  Write expression for maximum and minimum value of swaying couple.

14.	What is meant by vibration isolation and give some vibration isolating materials.

15.	What is torsional vibrations?  Give expression for natural frequency of free torsional vibration.

PART – C (5 x 15 = 75 MARKS)

16.	In a slider crank mechanism, the length of the crank and connecting rod are 150 mm and 600 mm respectively. The crank position is 60 º from the inner dead centre. The crank shaft speed is 450 r.p.m.(clockwise).  Using analytical method determine (i)velocity and acceleration of slider and (ii) angular velocity and angular acceleration of connecting rod.

(OR)

17.	The crank and connecting rod of the petrol engine, running at 1800 r.p.m. are 50 mm and 200 mm respectively. The diameter of the piston is 80 mm and the mass of the reciprocating parts is 1 kg. At a point during power stroke, the pressure on the piston is 0.7 N/mm2.When it has moved 10 mm from the inner dead centre. Determine (i) net load on the gudgeon pin (ii)thrust in the connecting rod (iii)reaction between the piston and cylinder and (iv) engine speed at which the above values becomes zero.

18.	The turning moment diagram for a multi cylinder engine has been drawn to a scale of 1 mm to 500 N-m torque and 1 mm to 6 deg of the crank displacement. The intercepted areas between output torque curve and mean resistance line taken in order from one end in Sq.mm are -30, +410, -280, +320, -330,  +250,  -360, +280, -260 sq.mm, when the engine is running at 800 r.p.m. The engine has a stroke of 300 mm and the fluctuation of speed is not to exceed ±2 % of the mean speed.  Determine the suitable diameter and the cross section of the flywheel rim for a limiting value of the safe centrifugal stress of 7 MPa. The material density may be assumed as 7200 kg/m3.The width of the rim is to be 5 times thickness.

(OR)

[P.T.O]

19.	A certain machine requires a torque of (5000 + 500 sinθ) Nm. to drive it, where the θ is angle of rotation of shaft measured from certain data. The machine is directly coupled to an engine which produces a torque of (5000 + 600 sin2θ) Nm. The flywheel and the other rotating parts attached to the engine has a mass of 500 kg at a radius of gyration of 0.4 m. If the mean speed is 150 r.p.m.  Find  (i) the fluctuation energy (ii) the total percentage of fluctuation of speed and (iii) maximum and minimum angular acceleration of the flywheel and the corresponding shaft position.

# 20.	Four masses m1, m2, m3 and m4 are 200 kg, 300 kg, 240 kg and 260 kg respectively. The corresponding radii of rotation are 0.2 m, 0.15 m, 0.25 m and 0.3 m respectively and the angles between successive masses are 45º, 75º and 135º respectively.  Find the position and magnitude of the balance mass required, if its radius of rotation is 0.2 m

(OR)

21.	An inside cylinder locomotive has its cylinder centre lines 0.7 m apart and has a stroke of 0.6 m. The rotating masses per cylinder are equivalent to 150 kg at the crank pin, and the reciprocating masses per cylinder are equivalent to 180 kg. the wheel centre lines are 1.5 m apart .The cranks are at right angles. The whole of the rotating and 2/3 of the reciprocating masses are to be balanced by masses 	placed at a radius of 0.6 m. Find the magnitude and direction of the balancing masses. Find the fluctuation in rail pressure under one wheel, variation of tractive effort and the magnitude of swaying couple at a crank speed of 300 r.p.m.

22.	The mass of single degree damped vibrating system is 7.5 kg and makes 24 free oscillations in 14 seconds when disturbed from its equilibrium position. The amplitude of vibration reduces to 0.25 of its initial value after 5 oscillations. Determine (i) stiffness of spring (ii) logarithmic decrement and

(iii) damping factor.

(OR)

23.	A mass of 10 kg is suspended from one end of helical spring the other end being fixed. The stiffness of the spring is 10 N/mm. The viscous damping causes the amplitude to decrease to 1/10th of the initial value in four complete oscillations. If a periodic force of 150 cos 50t N is applied at the mass in vertical direction, find the amplitude of the forced vibration. What is its value of resonance?

24.	A steel shaft ABCD 1.5 m long has flywheel at its ends A and D. The mass of the flywheel A is 600 kg and has a radius of gyration of 0.6 m. The mass of the flywheel D is 800 kg and has a radius of gyration of 0.9 m. The connecting shaft has a diameter of 50 mm for the portion AB which is 4 m long, and has a diameter of 60 mm for the portion BC which is 0.5 m long, and has a diameter of d	mm for the portion CD which is 0.6 m long. Determine (i) the diameter d for the portion CD so that the node of the torsional vibration of the system will be at the centre of the length BC (ii) the natural frequency of torsional vibrations. The modulus of rigidity of the shaft material is 80 GN/mm2.

(OR)

25.	Determine the natural frequencies of torsional oscillations for the following system. The system is a reciprocating IC engine coupled to a centrifugal pump through a pair of gears. The shaft from the flywheel of the engine to the gear wheel is of 60 mm diameter and 950 mm length. The shaft from the pinion to the pump is of 40 mm diameter and 300 mm length. The engine speed is 1/4th of the pump speed.

Moment of Inertia of the flywheel is 800 kg m2

Moment of Inertia of the gearwheel is 15 kg m2

Moment of Inertia of the pinion is 4kg m2

Moment of Inertia of the pump is 17 kg m2

The modulus of rigidity of the shaft material is 84 GN/mm2.