# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	DESIGN OF MACHINE ELEMENTS 					       Time: 3 hours

# Subject Code:	ME257		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

#### Use of design data book permitted.  Assume relevant data if required

PART – A (10 x 1 = 10 MARKS)

1.	Define Poisson’s ratio.

2.	What are the factors that affect endurance limit?

3.	The longest leaf in a leaf spring is called as ____________.

4.	Define fatigue limit.

5.	Name the types of keys.

6.	Define critical speed of a shaft.

7.	What is the use of caulking?

8.	Give any two applications of cotter joint.

9.	The usual cross-section of the arms of the flywheel are ____________.

10.	What is the use of crank shaft?

PART – B (5 x 3 = 15 MARKS)

11.	State the factors affecting the design process.

12.	Discuss about the types of cyclic stresses.

13.	Design a shaft to transmit 20 kW at 1000 rpm. Assume the allowable shear stress of the shaft material as 40 MPa.

14.	How does cotter joint differ from knuckle joint?

15.	What are the types of crank shaft?

PART – C (5 x 15 = 75 MARKS)

16.	An I Section beam of depth 250 mm is supported at the ends, 4 m apart. It is loaded by a weight of 4 kN, falling through a height ‘h’ and striking the beam at mid-span. Moment of inertia of the section is 8 X 107 mm4. Modulus of elasticity is 210 kN/mm2. Determine the value of ‘h’, if the allowable bending stress is 120 N/mm2.

(OR)

17.	A bolt is subjected to an axial force of 10000 N with a transverse shear force of 5000 N. Find the diameter of the required bolt according to, (a) Maximum principal stress theory (b) Maximum shear stress theory. The permissible tensile stress at elastic limit is 100 N/mm2 and Poisson’s ratio is 0.3.

18.	Find the diameter of a shaft to transmit twisting moments varying from 500 Nm to 2000 Nm. The ultimate tensile stress is 600 N/mm2, yield stress is 450 N/mm2. Assume stress concentration factor as 1.2, surface finish factor as 0.8, size factor as 0.85 and factor of safety as 2.

(OR)

19.	Design a helical compression spring to carry a load of 1.5 kN with a deflection of 40 mm. Spring index is 5. Allowable shear stress is 400 N/mm2. Modulus of rigidity is 8 X 1010 N/m2.

[P.T.O]

20.	A shaft running at 1000 rpm transmits 20 kW. It carries a central load of 1 kN. It is supported in bearings and the distance between their centers is 1 m. The permissible shear stress is 45N/mm2 and permissible bending stress is 70 N/mm2. Determine the diameter of the shaft. Assume minor shock loading.

(OR)

21.	A rigid flange coupling is to be designed to transmit 20 kW at 1000 rpm. The material of the shaft, key and bolt is mild steel and the material of the coupling is cast iron. The allowable shear stress for shaft and key is 40 N/mm2; Allowable crushing stress for key and bolt is 80 N/mm2; and allowable shear stress for coupling is 15 N/mm2. Design the coupling.

22.	A triple riveted butt joint with unequal cover plates is used to connect two 16 mm plates of a boiler. Design the joint completely, if the allowable stresses are 50 N/mm2, 40 N/mm2 and 80 N/mm2 in tension, shear and crushing respectively, for the plate and rivet material. Also find the efficiency of the joint.

(OR)

23.	Design a knuckle joint for transmitting an axial load of 60 kN. The stress in tension is 60 MPa, the stress in compression is 75 MPa and the stress in shear is 40 MPa.

24.	Design a cast iron piston for a single acting four stroke IC engine for the following specifications. Cylinder bore = 100 mm; Stroke length = 120 mm; Maximum gas pressure = 6 MPa; Brake mean effective pressure = 0.7 MPa; Fuel consumption = 0.24 kg/kW/hr;

Speed = 2200 rpm.

(OR)

25.	The flywheel of a punching machine provides 2600 Nm of energy, to punch a hole. The flywheel is 1.25 m in mean diameter and rotates at 150 rpm. Determine the cross-sectional area required for the rim of the cast-iron flywheel, if the co-efficient of fluctuation of speed is 0.15.