# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	MECHANICAL SYSTEMS FOR FOOD PROCESSING	       Time: 3 hours

# Subject Code:	ME262		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The principal advantage of radial piston pump is its _________.

2.	The principal element of vane pump is the _________.

3.	Length of a crossed belt drive depends upon _________ of the radii.

4.	_________ gears give velocity ratio of about 300:1.

5.	Lancashire boiler is a _________ pressure boiler.

6.	_________ is used only in stationary low pressure boiler.

7.	A _________ is provided for water removal in pressure type water cooler.

8.	The capacity of bottle type water cooler is approximately _________ litres.

9.	_________ is the mass that is to be handled at one time.

10.	A crane invariably includes a _________.

PART – B (5 x 3 = 15 MARKS)

11.	What are the types of casing adopted in a centrifugal pump?

12.	State the conditions for the transmission of maximum power in belt drives.

13.	Mention any three boiler mountings.

14.	Where is ice extensively used?

15.	What should systems integration encompass?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Describe the working principle, advantages and disadvantages of Peristaltic pump.

b.	Compare the merits and demerits of centrifugal and reciprocating pumps.

(OR)

17.	A centrifugal pump has the following dimensions: inlet radius=80mm; outlet radius=160mm; width of impeller at the inlet=50mm; ß1=0.45radians; ß2=0.25radians; width of impeller at outlet=50mm. Assuming shockless entry, determine the discharge and the head developed by the pump when the impeller rotates at 90 radians / second.

18.	A belt embraces the shorter pulley by an angle of 165o and runs at a speed of 1700 m/min. Dimensions of the belt are: Width=20 cm and thickness = 8mm. Its density is 1gm/cm3.  Determine the maximum power that can be transmitted at the above speed, if the maximum permissible stress in the belt is not to exceed 250N/cm2 and µ=0.25.

(OR)

19.	A solid shaft of mild steel 240mm diameter is to be replaced by a hollow shaft of alloy steel for which allowable shear stress is 22 percent greater. The power to be transmitted is to be increased by 20 percent and the speed of rotation increased by 5 percent. Determine the maximum internal diameter of the hollow shaft.

20.	Explain the constructional features and functioning of a Babcock and Wilcox boiler.

(OR)

[P.T.O]

21.	Distinguish between fire tube and water tube boilers. Explain the treatment of water for boilers.

22.	A refrigerator using NH3 works between the temperatures -100C and 250C.The gas is dry at the end of compression and there is no undercooling of liquid. Calculate the theoretical C.O.P. of the cycle. The properties of NH3 are given below:

| Temperature (0C) | Liquid heat (hf) (kJ/kg) | Liquid heat (hfg) (kJ/kg) | Liquid entropy(sf) (kJ/kg) |
| --- | --- | --- | --- |
| 25 -10 | 100.8 -33.7 | 1234.8 1352.5 | 0.349 -0.139 |

(OR)

23.	With the aid of a schematic diagram, explain the working of an ice plant.

24.	Explain the factors that influence the design of the material handling system.

(OR)

25.	Discuss in detail the conveyor operations and features.