# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2011

# Subject Title:	FINITE ELEMENTS ANALYSIS 						 Time: 3 hours

# Subject Code:	ME346			         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the various steps involved in solving a problem using finite element method.	(10)

b.	Explain the various engineering applications of finite element method and mention the 	field variable in each application. 						     						(10)

2.	The differential equation of a physical phenomenon is given by

By using a trial function, y = a1 (x – x3) + a2 (x –x5), calculate the value of the parameters a1 and a2 by the following methods, (i) Least Squares;	 (ii) Galerkin. The boundary conditions are: y (0) = 0, y (1) = 1.

(OR)

3.	a.	How do you generate an iso-parametric quadrilateral element for C1 continuity? Explain 	with example. 								     								(10)

b.	Explain the natural coordinate system for one and two dimensional elements. 	     	(10)

4.	The stresses induced in the axially loaded steeped bar of 200 kN is applied at point P as shown in Figure. Assume the following data A1 = 2400 mm2, E1=70 x 109 N/m2, A2=600 mm2, E2=200 x109 N/m2.

Calculate the following:

- The nodal displacement at point P		b.	Stress in each material

c.	Reaction force.

(OR)

5.	For a tapered plate of uniform thickness t=10 mm as shown in Figure, find the displacements at nodes by forming into two element model. The bar has mass density =7800Kg/m3, E= 2 x 105   MN / m2. In addition to self weight, the plate is subjected to a point load P= 10 kN at its center. Also determine the reaction force at the support.

[P.T.O]

6.	a.	Derive the lumped mass matrix for one dimensional bar element. 		     			(10)

b.	Derive the stiffness matrix for a 2-dimensional frame element. 			     			(10)

(OR)

7.	Derive the governing equation for the torsion problem. A solid prismatic shaft is shown in Figure subjected to a twisting moment Mz.

8.	Find the temperature distribution in the one dimensional fin is as shown in Figure.

(OR)

9.	Find the velocity distribution along the vertical centre line CD as shown in Figure 5. Due to symmetry, consider only portion ABCDEA in the finite element analysis. The boundaries AED and BC and be seen to be streamlines. Assume the value of the streamline along AED is zero as a reference value, because the velocity component entering into the face AB is constant.