# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2011

# Subject Title:	IRRIGATION ENGINEERING   						       Time: 3 hours

# Subject Code:	09CE235	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Consumptive use.

2.	Write the crop seasons.

3.	Define subsurface irrigation.

4.	State the limitations of sprinkler irrigation.

5.	Define aquifer.

6.	What is the transmissibility of the aquifer?

7.	List the theories of canal design.

8.	What is meant by regime channel?

9.	Define barrage.

10.	What is meant by canal escape?

PART – B (5 x 3 = 15 MARKS)

11.	List the various factors that influence the water requirements of crops.

12.	Recommend a suitable method of irrigation for a hilly terrain. Justify your answer.

13.	Draw the typical cross section of tube well.

14.	How are canals classified?

15.	What are the different types of cross drainage works commonly in use?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Discuss the advantages and disadvantages of irrigation.							(7)

b.	A water course has a culturable commanded area of 1200 hectares. The intensity of irrigation for crop A is 40% and for crop B is 35%, both the crop being rabi crops. Crop A has a kor period of 20 days and crop B has a kor period of 15 days. Calculate the discharge of the water course, if the depth for crop A is 10cm and for crop B is 16cm.			(8)

(OR)

17.	What is meant by irrigation efficiency? Explain various efficiencies under which irrigation performance is evaluated and mention how each one could be improved.

18.	a.	Describe in detail about drip irrigation, with its advantages.						(8)

b.	Explain the methods available to calculate average rainfall over a basin.          		(7)

(OR)

19.	a.	Describe the various methods of surface irrigation.								(8)

b.	What is lift irrigation? Explain the methods of lifting water from the source.   		(7)

20.	How is the measurement of yield of an open well done? Explain in detail.

(OR)

21.	What are the causes and effect of water logging?  How can water logging be controlled?

[P.T.O]

22.	a.	Describe the design procedure of a lined canal.									(8)

b.	Briefly explain how the alignment of canal is done.								(7)

(OR)

23.	Explain in detail the following terms with reference to a canal section				(4+3+4+4)

a.	Cross section and components		b.	Balancing depth		c.	 Canal maintenance

d.	Canal lining.

24.	a.	Briefly explain the concrete gravity dam with neat sketch and discuss the forces acting in 	gravity dams.                                                                                           				(8)

b.	Briefly explain the aqueduct with neat sketch.                                                     		(7)

(OR)

25.	Explain the various types and classifications of river training works with neat sketches.