# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2011

# Subject Title: 	APPLIED ELASTICITY AND PLASTICITY			             Time : 3 hours

# Subject Code: 	09CE302		         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	The state of stress at a point with respect to the xyz system is   3  2-2   2  0-1-2-1  2 kN/m2.  Determine the stress tensor relative to the x’y’z’ coordinates system obtained by a rotation through 30º about the z-axis.													(6)

b. 	The state of stress at a particular point relative to the xyz coordinate system is given by the stress matrix   1510-10   1010    0-10 0   40 MPa.  Determine the normal stress and the magnitude and direction of the shear stress on a surface intersecting the point and parallel to the plane given by the equation 2x-y+3z= 9.											(14)

2.	Deduce 2D polynomial expression for deflection due to bending of a cantilever loaded with tip point load at the free end.

(OR)

3.	a. 	Prove that the following are Airy’s stress functions and examine the stress distribution 	represented by them: (a) Φ = Ax2 + By2; (b) Φ = Ax3 and (c) Φ = A (x4 - 3x2y2).		(14)

b.	Show that the Airy’s stress function Φ = represents stress distribution in a cantilever beam loaded at the free end with load P.  Find the value of A if τxy = 0 at

y = ±  where b and h are width and depth respectively of the cantilever.			(6)

4.	Derive the expression for σr and σθ for the following cases (a) Rotating disc and cylinder        (b) Thin hollow disc of external radius ‘b’ and internal radius ‘a’ (c) Thin solid disc of radius ‘b’.

(OR)

5.	a.	A steel gun barrel is subjected to an internal pressure of 70 MPa.  The internal diameter of 	the barrel is 75 mm and external diameter of 225 mm.  A steel band 25 mm thick and 	internal diameter 0.075 mm smaller than the external diameter of the gum barrel is shrunk 	on to the gun barrel.  Calculate (a) the shrinkage pressure on the gun barrel (b) maximum 	stress in the steel band, and (c) minimum temperature to which the band must be heated to 	make the assembly.  For steel E = 200 GPa, ν =0.3 and coefficient of thermal expansion = 	10 x / ºC.																(12)

b. 	A cast iron shaft 500 mm diameter is rotating at 3000 rev/min.  Its density is 7470 kg/m3 and Poisson’s ratio 0.28.  It is constrained at its ends so that it cannot expand or contract axially.  Calculate the total longitudinal thrust over its entire cross section due to centrifugal stresses.															(8)

[P.T.O]

6.	a. 	A hollow circular torsion member has an outside diameter of 22 mm and inside diameter of 	18 mm, with mean diameter D = 20 mm and t/D = 0.10.  Calculate the torque and angle of 	twist per unit length if shearing stress at mean diameter is 70 MPa.  Calculate these values 	if a cut is made through the wall thickness along the entire length.  G = 77.5 GPa.	(5)

b. 	Derive the Poisson’s equation for torsion of prismatic bar of non circular cross section using St. Venant’s theory.														(15)

(OR)

7.	a. 	Derive an expression for torsion of a thin rectangular section using membrane analogy 	theory.																		(15)

b. 	A square shaft rotating at 250 rpm, transmits torque to a crane which is designed to lift maximum load of 150 kN at a speed of 10 m/min.  If the efficiency of crane gearing is 65%, estimate the size of the shaft for the maximum permissible shear stress of 35 MPa.  Also calculate the angle of twist of the shaft for a length of 3 m.  Take G = 100 GPa.

(5)

8.	Determine the diameter ‘d’ of a circular shaft subjected to a bending moment ‘M’ and a torque ‘T’, according to (i)  Maximum strain theory (ii) Octahedral shear stress theory (iii) Maximum energy theory  (iv) Maximum shear stress theory		  								(4x5)

(OR)

9.	a. 	Obtain an expression for yield surfaces of Tresca and Von Mises theory.			(10)

b. 	Derive Prandtl-Reuse equation for idealized plastic solid.							(10)