# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2011

# Subject Title:	MATERIAL SCIENCE 								     	       Time: 3 hours

# Subject Code:	EE223		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The first theory to explain the electrical conduction in conducting materials is the _______.

2.	Addition of impurities _______ the superconducting property.

3.	The electrical conductivity in semiconductor is produced due to movement of the _______ and _______.

4.	_______ effect is used to find whether the given semiconductor is n-type or p-type.

5.	In antiferromagnetic material, the dipoles are _______ in magnitude.

6.	The electro optic devices are using _______ materials.

7.	The materials which transmit light with negligible absorption and reflection are known as _______ materials.

8.	The excitation arises due to the absorption of photon and de-excitation of electrons with the release of radiation is called _______.

9.	_______ materials are used for a surgical application such as a blood vessel prostheses and heart valves.

10.	Write any one name of shape memory alloy materials.

PART – B (5 x 3 = 15 MARKS)

11.	Define mobility and give its unit.

12.	Define Fermi level in the case of semi conductors.  Mention its position in intrinsic and extrinsic semiconductors at 0K.

13.	What is the magnetic principle that is used in computer data storage?

14.	What is meant by traps and trapping centre?

15.	Explain the fundamental characteristics of shape memory alloys.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Deduce the mathematical expression for the electrical conductivity of a conducting 	material and hence obtain Wiedemann – Franz law.								(12)

b.	Calculate the electrical and thermal conductivities for a metal with a relaxation time 10-14 second at 300K (Density of electrons = 6 x 1028 m-3).								(3)

(OR)

17.	a.	Explain superconducting phenomena.  What are its properties?  Distinguish between type I 	and type II superconductors.													(12)

b.	Explain the applications of superconducting materials.							(3)

18.	a.	Write briefly, about the temperature dependence of carrier concentration and Fermi level in 	an extrinsic semiconductor.													(12)

b.	The conductivity of  Ge at 20 C is 2 -1 m-1.  What is its conductivity at 40 C?

Eg = 0.72eV.																	(3)

(OR)

[P.T.O]

19.	a.	What is Hall coefficient?  Derive an expression for Hall coefficient.  Describe the 	experimental set up for the measurement of the same.								(12)

b.	The Hall coefficient of certain silicon was found to be -7.35 x 10-5 m3 C-1 from 100 to 400K.  Determine the nature of the semiconductor.  If the conductivity was found to be 200 m-1-1, calculate the density and mobility of the charge carrier.					(3)

20.	a.	What is ferromagnetism?														(2)

b.	Explain the reason for the formation domain structure in ferromagnetic materials and how 	the hysteresis curve is explained on the basis of the domain theory.					(10)

c.	A paramagnetic material has a magnetic field intensity of 104 A/m.  If the susceptibility of the material at room temperature is 3.7 x 10-3, calculate the magnetization and flux density of the material.																(3)

(OR)

21.	a.	Derive the relation between the dielectric constant and atomic polarizability using internal 	field.																		(10)

b.	What is meant by dielectric breakdown?  Explain its types.						(5)

22.	a.	Discuss about absorption of light by semiconductors.								(5)

b.	Describe the different colour centres present in a crystal and their applications.		(5)

c.	Define the terms phosphorescence and fluorescence.								(5)

(OR)

23.	a.	Explain with a neat sketch the principle of photoconductivity.						(10)

b.	Explain with a neat sketch the working of photo detectors.							(5)

24.	a.	What are nano phase materials?												(3)

b.	What are the different methods used for the synthesis of nano phase materials.		(6)

c.	Write the properties of nano phase materials.										(6)

(OR)

25.	a.	What are advanced ceramics?  What are their characteristics?						(10)

b.	What is nonlinear optical effect?  Explain with a suitable example the non linear effect in a material.																		(5)