# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2011

# Subject Title : 	ELECTRIC CIRCUITS									      Time : 3 hours

# Subject Code:	EE245		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Kirchhoff’s voltage law.

2.	What are the types of controlled sources?

3.	Write an expression for transient current in a series RC circuit.

4.	Define Power Factor.

5.	What is loop analysis?

6.	What is meant by dependent and independent sources?

7.	State reciprocity theorem.

8.	State superposition theorem.

9.	Define Q factor.

10.	The resonant frequency of a series RLC circuit is _____________

PART – B  (5 x 3 = 15 MARKS)

11.	Obtain the delta connected equivalent for the star connected circuit given in Figure.

12.	A sine wave has a peak value of 25 V. Determine the following values:

a. 	rms                      b. 	peak to peak              c. 	average

13.	Define node voltages and mesh currents.

14.	Convert the given current source into its equivalent voltage source.

15.	The Q factor of a series circuit at 50 Hz is 2. If the circuit is supplied with constant voltage and variable frequency, determine the frequency at which the capacitor is at maximum voltage.

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Calculate the equivalent resistance Rab for the circuit given in figure.				(7)

b. 	For the network shown in Figure, find the branch currents through 6  resistor.		(8)

(OR)

17.	Using star delta transformation, obtain the equivalent resistance Rab for the circuit shown in figure, and also determine current ‘i’.

18.	a. 	A series circuit of R = 10, Xc = 5 has an effective applied voltage of 120 V. Determine 	the real power, reactive power, power factor.										(6)

b. 	A circuit having a resistance of 12  and inductance of 0.15H and a capacitance of 100 µF in series is connected across a 100V, 50Hz supply. Calculate the impedance, current, voltage across R, L, C and the phase difference between the current and supply voltage.																				(9)

(OR)

19.	A voltage of 230V is applied to a series RC circuit of R=8 and C=500µF. Determine the current, real power, power factor, reactive power, voltage across resistance, voltage across capacitance. Also draw the phasor diagram.

[P.T.O]

20.	Using nodal analysis, find the current in all branches of the circuit given in figure.

(OR)

21.	Determine the power dissipation in the 4ohm resistor of the circuit shown below using mesh analysis

3

5

2

6

4

60V

20V

-

+

+

-

22.	Determine the load resistance to receive maximum power from the source. Also find the maximum power delivered to the load in the circuit given in figure.

(OR)

[P.T.O]

23.	Determine the current through the 2  resistor as shown in figure, using superposition theorem.

24.	a. 	A coil is represented by a series combination of L=50mH and R=15. Calculate the 	quality factor at 10 kHz.														(2)

b. 	Derive the expression for Q in terms of the energy stored in the circuit.				(8)

c. 	Compare series resonance circuit with parallel resonant circuit.					(5)

(OR)

25.	In figure, a maximum current of 0.1 A flows through the circuit when the capacitor is at 5 µF with a fixed frequency and a voltage of 5V. Determine the frequency at which the circuit resonates, the bandwidth, the quality factor Q, and value of resistance at resonant frequency.