# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2011

# Subject Title:	DIGITAL PRINCIPLES AND SYSTEM DESIGN 		       Time: 3 hours

# Subject Code:	09IT204		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Convert (196.062)10 to octal.

2.	State the duality principle.

3.	Draw the logic diagram of Half Subtractor.

4.	What are the methods available in HDL?

5.	Mention the uses of demultiplexer.

6.	Draw the Logic Diagram of 4:1 multiplexer.

7.	What is meant by Edge Triggered Flip Flop?

8.	Give the excitation table of a T flip-flop.

9.	What is primitive flow table?

10.	What is one hot state assignment?

PART – B (5 x 3 = 15 MARKS)

11.	Define the laws of Absorption and Involution.

12.	Design a combination circuit with three input variables that will produce a logic 1 output when more than one input variables are logic 1.

13.	Compare PROM, PLA and PAL.

14.	What is the difference between Moore and Mealy Circuit Models?

15.	Explain the Hazards in combinational circuits?

PART – C (5 x 15 = 75 MARKS)

16.	Reduce the following expressions:

a.														(4)

b.															(4)

c.	What is the value of b if ? 											(3)

d.	Convert (37.29)10 to octal and hexadecimal. 										(4)

(OR)

17.	Reduce the following functions using K - Map techniques and implement using basic gates

a.	f(A, B, C, D, E) =  ∑m(1, 4, 8, 10, 11, 20, 22, 24, 25, 26) + d(0, 12, 16, 17) 			(8)

b.	f(A, B, C, D) = π m(4, 5, 6, 7, 8, 12, 13) + d(1, 15) 								(7)

18.	a.	Design and explain the operation of binary to BCD converter with K-Map and logic 	diagram.																		(10)

b.	Write a short note on Hardware Description Languages.							(5)

(OR)

19.	Explain the building of a 24 - bit magnitude comparator using 4 -bit comparator.

[P.T.O]

20.	a.	Explain the logic diagram of 3 to 8 line decoder.									(7)

b.	How do you construct a 4 x 16 decoder with two 3 x 8 decoders. 					(8)

(OR)

21.	Implement the following function using PAL.

W (A, B, C, D) =∑ m (2, 12, 13)

X (A, B, C, D) = ∑m (7, 8, 9, 10, 11, 12, 13, 14, 15)

Y (A, B, C, D) = ∑m (0, 2, 3, 4, 5, 6, 7, 8, 10, 11, 15)

Z (A, B, C, D) = ∑m (1, 2, 8, 12, 13)

22.	Design a sequential circuit using JK flip-flop for the following state table [use state diagram]

| Present state | Next state |  | Output |  |
| --- | --- | --- | --- | --- |
| AB | X=0 | X=1 | X=0 | X=1 |
| 00 | 00 | 11 | 1 | 0 |
| 01 | 01 | 11 | 1 | 1 |
| 10 | 01 | 00 | 1 | 0 |
| 11 | 11 | 10 | 0 | 0 |

(OR)

23.	a.	Explain Serial in Serial out Shift Register. 										(7)

b.	Explain Serial in parallel out Shift Register. 										(8)

24.	Explain the classification of Race- Free State Algorithm.

(OR)

25.	Design an Asynchronous sequential circuit using SR latch with two inputs A and B and one output y. B is the control input which, when equal to 1, transfers the input A to output y. When B is 0, the output does not change, for any change in input.