# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2011

# Subject Title: 	ADVANCED THERMODYNAMICS					             Time : 3 hours

# Subject Code: 	09ME302		         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

A steady flow compressor for a gas turbine receives air at 1 bar and 15°C, which it compresses to 7 bar with an efficiency of 83%. Based on surroundings at 5°C determine (a) the change of availability and the work for isentropic compression. For the actual process evaluate (b) the change of availability and work done, (c) the change of availability of the surroundings and   (d) the irreversibility. Treat the gas as an ideal one, with the specific heat at constant pressure, Cp = 1.004 kJ/ kg K, and the ratio of specific heats, k = 1.4.

2.	a.	Derive Maxwell’s relations.

b.	In terms of the measurable properties of a simple compressible substance, Show that:

CpCv=k= βTβs

(OR)

3.	Derive the following equations:

a.	U = F – T ∂F∂TV = - T2 ∂FT∂TV

b.	Cυ= -T ∂2F∂T2v

4.	Determine the fugacity of pure water for the following cases:

Saturated vapor at 100oC,

Saturated liquid at 100oC,

Compressed liquid at 100oC, and 200 bar.

Superheated vapor at 100oC, and 0.5 bar.

Saturated vapor at 350oC.

Super-cooled vapor at 90°C, 1 bar, Assume ideal gas behavior

(OR)

5.	Determine the maximum Joule-Thomson inversion temperature in terms of the critical temperature Tc predicted by the

(a)  Vander Waals' equations

(b)  Redlich-Kwong equation

(c)  Dieterici equation

6.	Nitrogen at a pressure of 250 atm and a temperature of 400 K expands reversibly and         adiabatically in a turbine to an exhaust   pressure of 5 atm. The flow rate 1 kg/s.  Calculate the power output if  Nitrogen obeys the Redlich-Kwong equation of state, for  Nitrogen at 1 atm.

(OR)

[P.T.O]

7.	Liquid octane enters a reactor with 40% excess air, with both the fuel and air being at 298 K and 1 atm.  Determine the adiabatic flame temperature in the case of (a) steady state steady flow device, (b) a rigid closed system. Also determine the final pressure for case (b). Assume ideal gas behavior and that Octane (l) = 703 kg m–3.

8.	For a gas obeying the van der Waals equation of state, show that:

a.	  	cp- cυ= R1-2aυ-b2RTυ3

b.	∂cvdυT= T2 ∂2υ∂T2V = 0 to prove that cυis a function of temperature only

(OR)

9.	According to Berthelot, the temperature effect of the second virial coefficient is given by

B'T= bT- aT3  where a and b are constants. Show that according to Berthelot,

TinvTB= 3