# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2011

# Subject Title:	GAS DYNAMICS AND JET PROPULSION 				       Time: 3 hours

# Subject Code:	ME217		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

#### (Use of gas table is permitted)

PART – A (10 x 1 = 10 MARKS)

1.	Express the stagnation enthalpy in terms of static enthalpy and velocity of flow.

2.	Define Mach number.

3.	What is subsonic and supersonic flow with respect to Mach number?

4.	When is M* used instead of M?

5.	Give any two assumption regarding Fanno flow.

6.	Differentiate between adiabatic flow and diabatic flow.

7.	Define strength of shock wave.

8.	What you understanding by ‘Oblique shock’?

9.	Give the equation of thrust for a turbojet engine.

10.	Give any two differences between ram jet and pulse jet engine.

PART – B (5 x 3 = 15 MARKS)

11.	Explain Mach cone and Mach angle.

12.	How do the area and velocity vary in supersonic flow of nozzle and diffuser?

13.	Explain about the significance of Rayleigh flow with diagram.

14.	Write the Prandtl-Meyer relation for normal shock.

15.	Explain about ‘after burning’ in turbojet engines?

PART – C (5 x 15 = 75 MARKS)

16.	Air (γ = 1.4, R = 287.43 J/kg K) enters a straight axisymmetric duct at 300 K, 3.45 bar, and

150 m/s and leaves it at 277 K, 2.058 bar and 260 m/s. the area of cross section at entry is 500 cm2. Assuming adiabatic flow, determine i) Stagnation temperature, ii) Maximum velocity, iii) Mass flow rate and iv) Area of cross section at exit.

(OR)

17.	What is the effect of Mach number on compressibility? Prove for γ = 1.4

18.	A diffuser has exit to throat area ratio of 1.5 to 1. The inlet Mach number is 0.8. The initial pressure and temperature are 1 bar and 15o C.  Assuming the flow to be isentropic, calculate the following for the exit air, i) Exit pressure, ii) Exit temperature and iii) Exit Mach number.

(OR)

19.	Derive the expression for the area variation in terms of Mach number.

[P.T.O]

20.	Show that : .

(OR)

21.	The condition of a gas (γ = 1.3, R= 465 J/kg K) at the entry of a constant area duct are                P1 = 0.35 bar, T1 = 310 K, C1 = 60 m/s. The heat supplied in the combustion chamber is 4500 kJ/kg. Calculate at exit, i) Pressure, ii) temperature, iii) Mach number, iv) Velocity of the gas and v) pressure and temperature of the gas at sonic condition.

22.	The state of a gas (γ = 1.3, R= 0.469 kJ/kg K) upstream of a normal shock wave is given by the following data: Mx = 2.5, Px = 2 bar, Tx = 275 K. Calculate the Mach number, pressure, temperature and velocity of the gas downstream of the shock; Check the calculated values with those given in the gas tables.

(OR)

23.	The stagnation pressure and temperature of air at the entry of a nozzle are 5 bar and 450 K respectively. The exit Mach number is 2.1 where a normal shock occurs. Calculate the following quantities before and after the shock, i) Static temperatures, ii) Stagnation temperatures, iii) Static pressures, and iv) Stagnation pressure loss.

24.	Describe the working of supersonic ramjet engine with a neat sketch. List out its advantages and disadvantages.

(OR)

25.	An aircraft propeller flies at a speed of 440 kmph. The diameter of the propeller is 4.1 m and the speed ratio is 0.8. The ambient conditions of air at the flight altitude are T = 255 K and

P = 0.55 bar. Find the following: i) Thrust	 ii) Thrust power and iii) Propulsive efficiency.