# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2011

# Subject Title:	MECHANICAL SYSTEMS FOR FOOD PROCESSING	       Time: 3 hours

# Subject Code:	ME262		       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the application of a diaphragm pump?

2.	A centrifugal pump should be chosen in preference to a Reciprocating pump, when uniform torque is essential.  (True / False)

3.	Name any two advantages of a Muff coupling.

4.	In V-belt, belt touches the spindle

a.	at the bottom   b.	 Both bottom and sides   c.	at the sides only

d. 	could touch everywhere.

5.	How are boilers specified?

6.	What are smoke tube boilers?

7.	The brine temperature for ice making should be maintained between

a.	-12 to -17 •C		b.	-12   to -10 •C	c.	-12 to - 15 •C	d.	 -12   to -16 •C

8.	What is dry ice?

9.	Mention any two types of elevators.

10.	What are the advantages of SS conveyors?

PART – B (5 x 3 = 15 MARKS)

11.	What is the function of air vessels in a reciprocating pump?

12.	What are the requirements of a good coupling?

13.	What are high and low pressure boilers? Give examples.

14.	Why water is agitated with air during production of ice?

15.	What is the difference between a conveyor and an elevator?

PART – C (5 x 15 = 75 MARKS)

16.	Describe the construction and working of a centrifugal pump with a neat sketch.

(OR)

17.	Enumerate the applications of the following pumps in the food processing industry:

a.	Reciprocating pump		b.	Vane pump

18.	A shaft supported between two bearings carries an overhanging bevel gear at one end. The speed of the shaft is 600 rpm and the torque on the shaft is 2.2x106 N-mm.  The ultimate and yield strength of the shaft material are 280 N/mm2 and 135 N/mm2 respectively. The maximum bending moment is 4.36x106 N-mm. The combined shock and fatigue factors in bending and torsion are 1.5 and 1 respectively. Find the diameter of the shaft and the power transmitted.

(OR)

19.	Explain briefly about the following with a neat sketch:

a.	Flange coupling		b.	Open and crossed belt drives

[P.T.O]

20.	Describe any one mounting and accessory of an Industrial boiler, with a neat sketch.

(OR)

21.	With a neat sketch, describe the construction and working of a Babcock and Wilcox boiler.

22.	Explain briefly about the following:

a.	Storage water chillers			b.	Instant water chillers.

(OR)

23.	How are clear ice bars produced in ice plants?

24.	Explain briefly about the following with a neat sketch:

a.	Screw elevator		b.	Plastic conveyor

(OR)

25.	“Improper handling of materials in food plants will lead to spoilage and detoriation” – support the statement with essential supplements.