# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2011

# Subject Title:	ELECTRO MAGNETIC THEORY 						 Time: 3 hours

# Subject Code:	PH305			         		        				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

List the four fundamental Maxwells equation for the time varying electromagnetic field along with their derivations. Explain Maxwell’s contribution to these equations.

2.	a.	State Gauss's law. Define electric flux density. Prove that ∇.D=ρ.   				(10)

b.	Consider a constant E field in the y-direction. If a cube, dimension, L were positioned such that its faces are perpendicular to the x, y, and z axes, what is the flux through the cube? Do the calculation for each face separately.											(10)

(OR)

3.	a.	Define in words, the electric potential V at any point. Obtain an equation for the 	relationship 	between the electric field intensity E and V. Explain the significance of this 	equation.																	(10)

b.	Define electric dipole moment. An electric dipole of charge Q are separated by a distance d.  Prove that the absolute potential at a point P(r, θ, ϕ) is =p.ir4πε0r2, where p is the dipole moment.                            														(10)

4.	a.	State Biot-Savart’s Law. Determine the magnetic field at a distance d from an infinitely 	long straight wire in which flows a current I, using Biot-Savart’s law.	 			(10)

b.	Write a short note on magnetization in materials.  Obtain a relationship between the magnetic susceptibility and relative permeability of materials. 						(10)

(OR)

5.	a.	State Ampere's circuit law. Prove the following. (i) Magnetostatic field is not conservative. 		(ii) Magnetic monopole does not exist.                									(10)

b.	An infinitely long transmission line consisting of two concentric cylinders is placed along the z-axis. The inner conductor has radius ‘a’ and carries current ‘I’ while the outer conductor has inner radius ‘b’ and thickness ‘t’ and carries return current ‘-I’ . Determine H everywhere assuming that current is uniformly distributed in both conductors.  		(10)

6.	Discuss the propagation of plane electromagnetic wave in conducting medium. Derive the dispersion relation for a conducting medium.

(OR)

7.	A plane electromagnetic wave with its electric vector given by E=E0ei(ωt-k.r) incident obliquely on a lossless media. Prove the Snell’s laws of reflection and refraction.

8.	Obtain expression for the Lienard-Wiechart Potentials for a moving electron. Hence evaluate the electric and magnetic field intensities for an electron in uniform motion. Discuss the results in the limits of low and high velocities.

(OR)

9.	Write short note on oscillating electric dipole and deduce an expression for radiation from an oscillating dipole.