# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2011

# Subject Title : 	METROLOGY AND COMPUTER AIDED INSPECTION	      Time : 3 hours

# Subject Code:	PE227		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Normal distribution is also called as ________.

2.	Define accuracy.

3.	_______ is used to make the surface plates.

4.	Comparators are available with magnification from 1000:1 to________.

5.	The single triangle which shows the surface finish is ________.

6.	Ra value of air is ________.

7.	The helix angle of a screw thread is tanα=pπd.  (True / False).

8.	The maximum error identified in gear manufacturing is ________.

9.	________ is one of the light sources for interferometer.

10.	________ is general consideration, we use in ultrasonic frequencies.

PART – B  (5 x 3 = 15 MARKS)

11.	Define the term dynamic metrology.

12.	What is meant by Line and End measurement?

13.	Define tool lay.

14.	List the various elements of screw thread.

15.	Define vision system.

PART – C  (5 x 15 = 75 MARKS)

16.	Briefly explain Controllable Errors.

(OR)

17.	Explain Selection and Calibration of instruments.

18.	With a neat sketch, explain the working of the following comparators:

a.	Johansson Mikroator

b.	Sigma Mechanical comparator

(OR)

19.	Describe the working of Autocollimator and give its merits and demerits.

20.	Explain with an example how tool maker’s microscope can be used in a comparator.

(OR)

21.	Define the following terms:													(5x3)

a.	Straightness        			b.	Flatness

c.	Run out              			d.	Concentricity

e.	Surface texture

22.	Derive expression for measurement of thread angle in a screw thread.

(OR)

[P.T.O]

23.	Find the nominal diameter of roller which will rest in the tooth space, with its centre on the pitch line, of an 8 DP 20  pressure angle gear having 40 teeth. If there is a backlash allowance of 0.002 to 0.004 inch. normal to tooth flank, what will be the limits of diameter measurement over rollers of this size?

24.	Explain the working principle of LASER interferometer and give the verities of measurements.

(OR)

25.	With suitable sketches, explain the working principle of CMM. Mention the various applications.