# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2012

# Subject Title:	MECHANICS OF FLUIDS 								       Time: 3 hours

# Subject Code:	09CE202	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define the term capillarity.

2.	What is a Newtonian fluid?

3.	Distinguish between Steady flow and unsteady flow.

4.	Write down the Euler’s Equation of motion.

5.	Name the flow measuring instruments which use the Bernoulli’s equation.

6.	Define Cippoletti Weir.

7.	What is meant by equivalent pipe?

8.	Enlist minor losses in flow through pipes.

9.	Define Similitude.

10.	What is meant by Dimensional Homogenity?

PART – B (5 x 3 = 15 MARKS)

11.	Determine Mass density, Specific volume, and Specific weight of liquid whose specific gravity is 0.85.

12.	Differentiate free and forced vortex flow with examples.

13.	Explain Reynolds’s experiment.

14.	Explain Water hammer and state the two conditions under which water hammer occur.

15.	Obtain Scale ratio for velocity, time for the models governed by equality of Froude’s number.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Define Specific gravity and Viscosity of fluid.				                    				(5)

b.	The space between two square flat plate is filled with oil.   Each side of the plate is 60cm.   The thickness of the oil film is 12.5 mm, the upper plate which moves at 2.5 m/sec requires a force of 10 kgf to maintain the speed. Determine dynamic viscosity of oil in poise, Kinematic viscosity of oil in stokes, if the specific gravity of oil is 0.95. 							(10)

(OR)

17.	a.	Find the surface tension of soap bubble of 40 mm, when the inside pressure of the bubble 	2.5N/m2above atmospheric pressure.						         					(5)

b.	A U- tube Manometer is used to measure the pressure of oil of specific gravity 0.85 flowing in a pipe and the right limb is open to atmosphere. The centre of pipe is 100 mm below the level of mercury in the open limb. If the difference in mercury level in two limbs is 160 mm, find the absolute pressure of oil in the pipe.			       									(10)

18.	a.	Explain briefly the different types of flow					        					(8)

b.	The stream function is given by ψ =5x-6y, Calculate the velocity components and also magnitude and direction of the resultant velocity at any time.			        			(7)

(OR)

[P.T.O]

19.	a.	If for a two dimensional potential flow the velocity potential is given by Φ = x (2y-1), 	Determine the velocity at point p (4, 5). Determine also the value of stream function at that 	point.										       									(8)

b.	Derive Euler’s equation of motion and hence deduce Bernoulli’s equation             		(7)

20.	a.	Derive Darcy’s Weisbach Equation.					        						(7)

b.	A horizontal venturimeter with inlet and throat diameter 300 mm and 100mm respectively is used to measure the rate of flow. The pressure intensity at the inlet is 130 kN/m2 and the vacuum pressure at the throat is 350mm of mercury. Assuming 3% head lost between inlet and throat, find the coefficient of discharge.				       							(8)

(OR)

21.	a.	Derive an expression for determination of discharge through an orifice meter.       		(7)

b.	Water flows over a rectangular sharp crested weir 1m long, head over the sill of the weir being 0.66 m. The approach channel is 1.4 m wide and the depth of the flow in the channel is 1.2 m. starting from principles. Determine the rate of discharge over the weir. Consider also the velocity of approach and the effect of end contractions. Take Coefficient discharge of weir as 0.6								        									(8)

22.	a.	Determine the rate of flow of water through the pipe of diameter 20 cm and length 50m with 	one end of the pipe is connected to a tank and the other end is open to atmosphere. The pipe 	is horizontal and the height of water in the tank is 4m above the centre of the pipe. Consider 	all the minor losses and take f = 0.009 			        								(8)

b.	Three pipes of length 800 m, 500 m, and 400 m and of diameter 500 mm, 400 mm, and 300 mm respectively connected in series. These pipes are to be replaced by a single pipe of length 1700 m. Find the diameter of the single pipe.				          						(7)

(OR)

23.	a.	The rate of flow of water through the horizontal pipe is 0.25m3/s. The diameter of the pipe 	changes from 200mm to 400 mm. The pressure intensity in the smaller pipe is 	11.772N/cm2.  Find  i) Loss of head due to sudden enlargement, ii) pressure intensity in 	large pipes.		         															(8)

b.	Write short notes on Energy gradient line and Hydraulic gradient line 		         		(7)

24.	Using Buckingham π Theorem, Show that the velocity through a circular orifice is given by

V = √2gH φ (D/H, μ /ρvH ) where, H=head, D=diameter of the orifice, μ=coefficient of viscosity, ρ = mass density, g= acceleration due to gravity.

(OR)

25.	a.	Water is flowing through a pipe of diameter 30 cm at a velocity of 4m/sec. Find the velocity 	of oil flowing in another pipe of diameter 10 cm. If the condition of dynamic viscosity of 	water and oil is given as 0.01 poise and 0.025 poise. The specific gravity of oil is 0.8.	       (8)

b.	List out the various types of model Laws and explain any three. 						(7)